
[image: image1.emf]


USER MANUAL

LED MOVING HEAD SPOT LIGHT

[image: image2.jpg]



[image: image3.wmf]
Please read over this manual before using the light

Maintaining

1. The fixture should be kept dry to avoid working in wet conditions.

2. Intermittent use will effectively prolong the life of this fixture.

3. For a good ventilation and lightiing effects,please clean the fans ,fan nets and lens usually .

4. To avoid damage this fixture, please don't use alcholol or other organic solvents to clean the fixture shell.

Declaration

Before delivery,this fixture

have the performance intact, packaging integrity, and all users should strictly observe the above warnings and operating instructions for the matter, with any damage caused by misuse is not guaranteed wihtin the tcompany,for damages caused by ignoring the munual instruction is not withing the scope of the distributors or dealers.Thank you for your cooperation!!

Precautions

1. In order to ensure the life of this fixture,pelase don't place this fixture where is moisture or leakage,and don't operate the fixture at temperatures higher than 40° please!

2. Don't place the fixture where is easy to loose or vibration-prone

3. To avoid risk of electric shock, please ask the professionals to maintain this fixture.

4. When connect the fixture to power, the changes of popwer votage should  be at-10% ~+10%,if higher, will reduce the life of the fixture, but if lower,will effect the lightness.

5. Should need 20 Min. to fulling cool the light before re-use.

6. In order to ensure the normal use of this product, please read this manual caresuly. Thank you very much for your cooperations.

Product Instruction

Working votage: AC220V/50Hz/ 110V/60Hz 
Power Consumption: 105W

Light Source: 27W LED

Channel Signal: DMX 512 6/12 channel

Palette: 7+white+rainbow effect

Gobo wheel: 7gobo+white+rainbow effect

Strobe: 0-10/s,Adjustable

Dimmer: 0%~100% Linear Dimming

Pan: 540°8bit/16bit adjustable, Electronic correction

Titl: 270°8bit/16bit adjustable, Electronic correction.

Special function: Built -in procedures for a variety of lighting effects,can run simultaneously from sound control without controllers

Optical System: Combination of high-precision quartz lens, so that optical loss is reduced to a minimum

Protection: All intelligent temperature thermal protection and high-interference with electronic circuits.
DMX channel Values(12 channel)
Our products accpet international standard DMX 512, as below: 
	Channel
	DMX values
	Functions

	1 Pan
	0~255
	0~540°

	2 Pan 16 Bit
	0~255
	16bit adjustable

	3 Tilt
	0~255
	0~270°

	4 Tilt  16 Bit
	0~255
	16bit adjustable

	Channel
	DMX values
	Functions

	5Pan/Tilt Speed
	0~255
	From fast to slow

	6 Strobe
	0~3
	off

	
	4~7
	open

	
	8~215
	Strobe efect from fast to slow

	
	216~255
	open

	7 Dimmer
	0~255
	From dark to light

	8.Color
	0~255
	6 + whtie

	9 Gobo Wheel
	0~255
	6+White

	10 Auto foucs
	0~255
	Gobo Rotation

	11.Focus
	
	

	12 Bulit-in Channel
	0~7
	No function

	
	8~23
	Bulit-in1

	
	24~39
	Bulit-in 2

	
	40~55
	Bulit-in 3

	
	56~71
	Bulit-in 4

	
	72~87
	Bulit-in 5

	
	88~103
	Bulit-in 6

	
	104~119
	Bulit-in 7

	
	120~135
	Bulit-in 8

	
	136~151
	Sound 1

	
	152~167
	Sound 2

	
	168~183
	Sound 3

	
	184~199
	Sound 4

	
	200~215
	Sound 5

	
	216~231
	Sound 6

	
	232~247
	Sound 7

	
	248~255
	Sound 8


DMX channel Values (6CH)

Our products accpet international standard DMX 512, as below: 
	Channel
	DMX values
	Functions

	1 Pan
	0~255
	0~540°

	2 Tilt
	0~255
	0~270°

	3 Strobe
	0~3
	off

	
	4~7
	open

	
	8~215
	Strobe efect from fast to slow

	
	216~255
	open

	4 Palette
	0~4
	White

	
	5~9
	Yellow

	
	10~14
	Pink

	
	15-19
	Green

	
	20~24
	Rose

	
	25~29
	Pale

	
	30~34
	Olivine

	
	35~39
	Orange

	
	40~44
	Blue

	
	45~56
	White-Yellow

	
	57~65
	Yellow-Pink

	
	66~74
	Pink-Green

	
	75~83
	Green-Rose

	
	84~92
	Rose~Pale

	
	93~101
	Pale- Olivine

	
	102~110
	Olivine -Orange

	
	111~119
	Orange –Blue

	
	120~127
	Blue –White

	
	128~191
	Forwardrainbow

From slow to fast

	
	192~255
	Revise rainbow

From slow to fast

	5 Gobo Wheel
	0~7
	White

	
	8~15
	Gobo 1

	
	16~23
	Gobo 2

	
	24~31
	Gobo 3

	
	32~39
	Gobo 4

	
	40~47
	Gobo 5

	
	48~55
	Gobo 6

	
	56~63
	Gobo 7

	
	64~71
	Gobo 7 shake effect

	
	72~79
	Gobo 6 shake effect

	
	80~87
	Gobo 5 shake effect

	
	88~95
	Gobo 4 shake effect

	
	96~103
	Gobo3 shake effect

	
	104~111
	Gobo 2 shake effect

	
	112~119
	Gobo 1 shake effect

	
	120~127
	White

	
	128~191
	Forward rainbow

From slow to fast

	
	192~255
	Revise rainbow

From slow to fast

	6 Auto foucs
	
	


Display Control Instruction

Code Channel

Professional users control panel features, as early as the initial interface hold down the “MODE” button 10 seconds, in the Display menu press “UP”,” DOWN” key to enter the 2323 password, enter the Debug menu

P128    Mean Pan adjustment

E128    Mean Tilt adjustment

G128    Gobo wheel adjustment

C128    Mean Paltte adjustment

All conditioning by “UP”,”DOWN” adjustment, press “ENTER” to determine,and automatically exit in 10 seconds
[image: image4.png]



�EMBED CorelDRAW.Graphic.14���





1

[image: image5.jpg]


_1337608686.unknown

