# Blackbody color datafile (bbr_color.txt)

# Mitchell Charity

# http://www.vendian.org/mncharity/dir3/blackbody/

# Version 2001-Jun-22

# History:

#   2001-Jun-22  Switched to sRGB from Rec.709.

#   2001-Jun-04  Initial release.

# Fields:

#  K     temperature (K)

#  CMF   {" 2deg","10deg"}

#          either CIE 1931  2 degree CMFs with Judd Vos corrections

#              or CIE 1964 10 degree CMFs

#  x y   chromaticity coordinates

#  P     power in semi-arbitrary units

#  R G B {0-1}, normalized, mapped to gamut, logrithmic

#        (sRGB primaries and gamma correction)

#  r g b {0-255}

#  #rgb  {00-ff}

#

1000 K   2deg  0.6499 0.3474  2.472e+06    1.0000 0.0337 0.0000  255  51   0  #ff3300

1000 K  10deg  0.6472 0.3506  2.525e+06    1.0000 0.0401 0.0000  255  56   0  #ff3800

1100 K   2deg  0.6361 0.3594  2.112e+07    1.0000 0.0592 0.0000  255  69   0  #ff4500

1100 K  10deg  0.6348 0.3612  2.162e+07    1.0000 0.0631 0.0000  255  71   0  #ff4700

1200 K   2deg  0.6226 0.3703  1.282e+08    1.0000 0.0846 0.0000  255  82   0  #ff5200

1200 K  10deg  0.6225 0.3710  1.314e+08    1.0000 0.0860 0.0000  255  83   0  #ff5300

1300 K   2deg  0.6095 0.3801  5.959e+08    1.0000 0.1096 0.0000  255  93   0  #ff5d00

1300 K  10deg  0.6104 0.3797  6.118e+08    1.0000 0.1085 0.0000  255  93   0  #ff5d00

1400 K   2deg  0.5966 0.3887  2.243e+09    1.0000 0.1341 0.0000  255 102   0  #ff6600

1400 K  10deg  0.5983 0.3874  2.306e+09    1.0000 0.1303 0.0000  255 101   0  #ff6500

1500 K   2deg  0.5841 0.3962  7.121e+09    1.0000 0.1578 0.0000  255 111   0  #ff6f00

1500 K  10deg  0.5864 0.3940  7.333e+09    1.0000 0.1515 0.0000  255 109   0  #ff6d00

1600 K   2deg  0.5720 0.4025  1.967e+10    1.0000 0.1806 0.0000  255 118   0  #ff7600

1600 K  10deg  0.5747 0.3996  2.028e+10    1.0000 0.1718 0.0000  255 115   0  #ff7300

1700 K   2deg  0.5601 0.4076  4.839e+10    1.0000 0.2025 0.0000  255 124   0  #ff7c00

1700 K  10deg  0.5632 0.4041  4.999e+10    1.0000 0.1912 0.0000  255 121   0  #ff7900

1800 K   2deg  0.5486 0.4118  1.081e+11    1.0000 0.2235 0.0000  255 130   0  #ff8200

1800 K  10deg  0.5519 0.4077  1.118e+11    1.0000 0.2097 0.0000  255 126   0  #ff7e00

1900 K   2deg  0.5375 0.4150  2.224e+11    1.0000 0.2434 0.0000  255 135   0  #ff8700

1900 K  10deg  0.5408 0.4104  2.305e+11    1.0000 0.2272 0.0000  255 131   0  #ff8300

2000 K   2deg  0.5267 0.4173  4.268e+11    1.0000 0.2647 0.0033  255 141  11  #ff8d0b

2000 K  10deg  0.5300 0.4123  4.431e+11    1.0000 0.2484 0.0061  255 137  18  #ff8912

2100 K   2deg  0.5162 0.4188  7.710e+11    1.0000 0.2889 0.0120  255 146  29  #ff921d

2100 K  10deg  0.5195 0.4134  8.018e+11    1.0000 0.2709 0.0153  255 142  33  #ff8e21

2200 K   2deg  0.5062 0.4196  1.322e+12    1.0000 0.3126 0.0219  255 152  41  #ff9829

2200 K  10deg  0.5093 0.4139  1.377e+12    1.0000 0.2930 0.0257  255 147  44  #ff932c

2300 K   2deg  0.4965 0.4198  2.166e+12    1.0000 0.3360 0.0331  255 157  51  #ff9d33

2300 K  10deg  0.4995 0.4138  2.260e+12    1.0000 0.3149 0.0373  255 152  54  #ff9836

2400 K   2deg  0.4872 0.4194  3.408e+12    1.0000 0.3589 0.0454  255 162  60  #ffa23c

2400 K  10deg  0.4900 0.4132  3.563e+12    1.0000 0.3364 0.0501  255 157  63  #ff9d3f

2500 K   2deg  0.4782 0.4186  5.178e+12    1.0000 0.3814 0.0588  255 166  69  #ffa645

2500 K  10deg  0.4808 0.4122  5.422e+12    1.0000 0.3577 0.0640  255 161  72  #ffa148

2600 K   2deg  0.4696 0.4173  7.624e+12    1.0000 0.4034 0.0734  255 170  77  #ffaa4d

2600 K  10deg  0.4720 0.4108  7.996e+12    1.0000 0.3786 0.0790  255 165  79  #ffa54f

2700 K   2deg  0.4614 0.4158  1.092e+13    1.0000 0.4250 0.0889  255 174  84  #ffae54

2700 K  10deg  0.4636 0.4091  1.147e+13    1.0000 0.3992 0.0950  255 169  87  #ffa957

2800 K   2deg  0.4535 0.4139  1.524e+13    1.0000 0.4461 0.1054  255 178  91  #ffb25b

2800 K  10deg  0.4555 0.4071  1.604e+13    1.0000 0.4195 0.1119  255 173  94  #ffad5e

2900 K   2deg  0.4460 0.4118  2.081e+13    1.0000 0.4668 0.1229  255 182  98  #ffb662

2900 K  10deg  0.4478 0.4050  2.193e+13    1.0000 0.4394 0.1297  255 177 101  #ffb165

3000 K   2deg  0.4388 0.4095  2.784e+13    1.0000 0.4870 0.1411  255 185 105  #ffb969

3000 K  10deg  0.4404 0.4026  2.939e+13    1.0000 0.4589 0.1483  255 180 107  #ffb46b

3100 K   2deg  0.4320 0.4070  3.657e+13    1.0000 0.5067 0.1602  255 189 111  #ffbd6f

3100 K  10deg  0.4333 0.4002  3.866e+13    1.0000 0.4781 0.1677  255 184 114  #ffb872

3200 K   2deg  0.4254 0.4044  4.725e+13    1.0000 0.5259 0.1800  255 192 118  #ffc076

3200 K  10deg  0.4266 0.3976  5.002e+13    1.0000 0.4970 0.1879  255 187 120  #ffbb78

3300 K   2deg  0.4192 0.4018  6.012e+13    1.0000 0.5447 0.2005  255 195 124  #ffc37c

3300 K  10deg  0.4202 0.3950  6.374e+13    1.0000 0.5155 0.2087  255 190 126  #ffbe7e

3400 K   2deg  0.4132 0.3990  7.545e+13    1.0000 0.5630 0.2216  255 198 130  #ffc682

3400 K  10deg  0.4140 0.3923  8.010e+13    1.0000 0.5336 0.2301  255 193 132  #ffc184

3500 K   2deg  0.4075 0.3962  9.349e+13    1.0000 0.5809 0.2433  255 201 135  #ffc987

3500 K  10deg  0.4082 0.3895  9.939e+13    1.0000 0.5515 0.2520  255 196 137  #ffc489

3600 K   2deg  0.4021 0.3934  1.145e+14    1.0000 0.5983 0.2655  255 203 141  #ffcb8d

3600 K  10deg  0.4026 0.3868  1.219e+14    1.0000 0.5689 0.2745  255 199 143  #ffc78f

3700 K   2deg  0.3969 0.3905  1.387e+14    1.0000 0.6153 0.2881  255 206 146  #ffce92

3700 K  10deg  0.3973 0.3841  1.479e+14    1.0000 0.5860 0.2974  255 201 148  #ffc994

3800 K   2deg  0.3919 0.3877  1.665e+14    1.0000 0.6318 0.3112  255 208 151  #ffd097

3800 K  10deg  0.3922 0.3813  1.776e+14    1.0000 0.6028 0.3207  255 204 153  #ffcc99

3900 K   2deg  0.3872 0.3849  1.979e+14    1.0000 0.6480 0.3346  255 211 156  #ffd39c

3900 K  10deg  0.3873 0.3786  2.115e+14    1.0000 0.6193 0.3444  255 206 159  #ffce9f

4000 K   2deg  0.3827 0.3820  2.333e+14    1.0000 0.6636 0.3583  255 213 161  #ffd5a1

4000 K  10deg  0.3827 0.3759  2.496e+14    1.0000 0.6354 0.3684  255 209 163  #ffd1a3

4100 K   2deg  0.3784 0.3793  2.729e+14    1.0000 0.6789 0.3823  255 215 166  #ffd7a6

4100 K  10deg  0.3783 0.3733  2.923e+14    1.0000 0.6511 0.3927  255 211 168  #ffd3a8

4200 K   2deg  0.3743 0.3765  3.169e+14    1.0000 0.6938 0.4066  255 217 171  #ffd9ab

4200 K  10deg  0.3741 0.3707  3.397e+14    1.0000 0.6666 0.4172  255 213 173  #ffd5ad

4300 K   2deg  0.3704 0.3738  3.654e+14    1.0000 0.7083 0.4310  255 219 175  #ffdbaf

4300 K  10deg  0.3701 0.3681  3.922e+14    1.0000 0.6817 0.4419  255 215 177  #ffd7b1

4400 K   2deg  0.3666 0.3711  4.188e+14    1.0000 0.7223 0.4556  255 221 180  #ffddb4

4400 K  10deg  0.3662 0.3656  4.500e+14    1.0000 0.6966 0.4668  255 217 182  #ffd9b6

4500 K   2deg  0.3631 0.3685  4.771e+14    1.0000 0.7360 0.4803  255 223 184  #ffdfb8

4500 K  10deg  0.3625 0.3631  5.132e+14    1.0000 0.7111 0.4919  255 219 186  #ffdbba

4600 K   2deg  0.3596 0.3659  5.406e+14    1.0000 0.7494 0.5051  255 225 188  #ffe1bc

4600 K  10deg  0.3590 0.3607  5.820e+14    1.0000 0.7253 0.5170  255 221 190  #ffddbe

4700 K   2deg  0.3563 0.3634  6.093e+14    1.0000 0.7623 0.5299  255 226 192  #ffe2c0

4700 K  10deg  0.3557 0.3583  6.567e+14    1.0000 0.7392 0.5422  255 223 194  #ffdfc2

4800 K   2deg  0.3532 0.3609  6.835e+14    1.0000 0.7750 0.5548  255 228 196  #ffe4c4

4800 K  10deg  0.3524 0.3560  7.373e+14    1.0000 0.7528 0.5675  255 225 198  #ffe1c6

4900 K   2deg  0.3502 0.3585  7.632e+14    1.0000 0.7872 0.5797  255 229 200  #ffe5c8

4900 K  10deg  0.3494 0.3538  8.240e+14    1.0000 0.7661 0.5928  255 227 202  #ffe3ca

5000 K   2deg  0.3473 0.3561  8.485e+14    1.0000 0.7992 0.6045  255 231 204  #ffe7cc

5000 K  10deg  0.3464 0.3516  9.170e+14    1.0000 0.7792 0.6180  255 228 206  #ffe4ce

5100 K   2deg  0.3446 0.3538  9.396e+14    1.0000 0.8108 0.6293  255 232 208  #ffe8d0

5100 K  10deg  0.3436 0.3494  1.016e+15    1.0000 0.7919 0.6433  255 230 210  #ffe6d2

5200 K   2deg  0.3419 0.3516  1.036e+15    1.0000 0.8221 0.6541  255 234 211  #ffead3

5200 K  10deg  0.3409 0.3473  1.122e+15    1.0000 0.8044 0.6685  255 232 213  #ffe8d5

5300 K   2deg  0.3394 0.3494  1.139e+15    1.0000 0.8330 0.6787  255 235 215  #ffebd7

5300 K  10deg  0.3383 0.3453  1.234e+15    1.0000 0.8167 0.6937  255 233 217  #ffe9d9

5400 K   2deg  0.3369 0.3472  1.248e+15    1.0000 0.8437 0.7032  255 237 218  #ffedda

5400 K  10deg  0.3358 0.3433  1.353e+15    1.0000 0.8286 0.7187  255 235 220  #ffebdc

5500 K   2deg  0.3346 0.3451  1.363e+15    1.0000 0.8541 0.7277  255 238 222  #ffeede

5500 K  10deg  0.3334 0.3413  1.479e+15    1.0000 0.8403 0.7437  255 236 224  #ffece0

5600 K   2deg  0.3323 0.3431  1.484e+15    1.0000 0.8642 0.7519  255 239 225  #ffefe1

5600 K  10deg  0.3311 0.3394  1.611e+15    1.0000 0.8518 0.7686  255 238 227  #ffeee3

5700 K   2deg  0.3302 0.3411  1.611e+15    1.0000 0.8740 0.7760  255 240 228  #fff0e4

5700 K  10deg  0.3289 0.3376  1.750e+15    1.0000 0.8630 0.7933  255 239 230  #ffefe6

5800 K   2deg  0.3281 0.3392  1.744e+15    1.0000 0.8836 0.8000  255 241 231  #fff1e7

5800 K  10deg  0.3268 0.3358  1.896e+15    1.0000 0.8740 0.8179  255 240 233  #fff0e9

5900 K   2deg  0.3261 0.3373  1.883e+15    1.0000 0.8929 0.8238  255 243 234  #fff3ea

5900 K  10deg  0.3247 0.3341  2.049e+15    1.0000 0.8847 0.8424  255 242 236  #fff2ec

6000 K   2deg  0.3242 0.3355  2.028e+15    1.0000 0.9019 0.8473  255 244 237  #fff4ed

6000 K  10deg  0.3228 0.3324  2.208e+15    1.0000 0.8952 0.8666  255 243 239  #fff3ef

6100 K   2deg  0.3223 0.3337  2.179e+15    1.0000 0.9107 0.8707  255 245 240  #fff5f0

6100 K  10deg  0.3209 0.3307  2.374e+15    1.0000 0.9055 0.8907  255 244 242  #fff4f2

6200 K   2deg  0.3205 0.3319  2.337e+15    1.0000 0.9193 0.8939  255 246 243  #fff6f3

6200 K  10deg  0.3190 0.3291  2.548e+15    1.0000 0.9156 0.9147  255 245 245  #fff5f5

6300 K   2deg  0.3188 0.3302  2.500e+15    1.0000 0.9276 0.9168  255 247 245  #fff7f5

6300 K  10deg  0.3173 0.3275  2.728e+15    1.0000 0.9254 0.9384  255 246 248  #fff6f8

6400 K   2deg  0.3171 0.3286  2.669e+15    1.0000 0.9357 0.9396  255 248 248  #fff8f8

6400 K  10deg  0.3156 0.3260  2.914e+15    1.0000 0.9351 0.9619  255 248 251  #fff8fb

6500 K   2deg  0.3155 0.3270  2.845e+15    1.0000 0.9436 0.9621  255 249 251  #fff9fb

6500 K  10deg  0.3140 0.3245  3.108e+15    1.0000 0.9445 0.9853  255 249 253  #fff9fd

6600 K   2deg  0.3140 0.3254  3.026e+15    1.0000 0.9513 0.9844  255 249 253  #fff9fd

6600 K  10deg  0.3124 0.3231  3.308e+15    0.9917 0.9458 1.0000  254 249 255  #fef9ff

6700 K   2deg  0.3125 0.3238  3.214e+15    0.9937 0.9526 1.0000  254 250 255  #fefaff

6700 K  10deg  0.3109 0.3217  3.515e+15    0.9696 0.9336 1.0000  252 247 255  #fcf7ff

6800 K   2deg  0.3110 0.3224  3.407e+15    0.9726 0.9395 1.0000  252 248 255  #fcf8ff

6800 K  10deg  0.3094 0.3203  3.728e+15    0.9488 0.9219 1.0000  249 246 255  #f9f6ff

6900 K   2deg  0.3097 0.3209  3.606e+15    0.9526 0.9270 1.0000  250 247 255  #faf7ff

6900 K  10deg  0.3080 0.3190  3.948e+15    0.9290 0.9107 1.0000  247 245 255  #f7f5ff

7000 K   2deg  0.3083 0.3195  3.811e+15    0.9337 0.9150 1.0000  247 245 255  #f7f5ff

7000 K  10deg  0.3066 0.3177  4.174e+15    0.9102 0.9000 1.0000  245 243 255  #f5f3ff

7100 K   2deg  0.3070 0.3181  4.022e+15    0.9157 0.9035 1.0000  245 244 255  #f5f4ff

7100 K  10deg  0.3053 0.3164  4.407e+15    0.8923 0.8897 1.0000  243 242 255  #f3f2ff

7200 K   2deg  0.3058 0.3168  4.238e+15    0.8986 0.8925 1.0000  243 243 255  #f3f3ff

7200 K  10deg  0.3040 0.3152  4.647e+15    0.8753 0.8799 1.0000  240 241 255  #f0f1ff

7300 K   2deg  0.3045 0.3154  4.460e+15    0.8823 0.8819 1.0000  241 241 255  #f1f1ff

7300 K  10deg  0.3028 0.3140  4.892e+15    0.8591 0.8704 1.0000  239 240 255  #eff0ff

7400 K   2deg  0.3034 0.3142  4.688e+15    0.8668 0.8718 1.0000  239 240 255  #eff0ff

7400 K  10deg  0.3016 0.3128  5.144e+15    0.8437 0.8614 1.0000  237 239 255  #edefff

7500 K   2deg  0.3022 0.3129  4.920e+15    0.8520 0.8621 1.0000  238 239 255  #eeefff

7500 K  10deg  0.3004 0.3117  5.402e+15    0.8289 0.8527 1.0000  235 238 255  #ebeeff

7600 K   2deg  0.3011 0.3117  5.159e+15    0.8379 0.8527 1.0000  236 238 255  #eceeff

7600 K  10deg  0.2993 0.3106  5.666e+15    0.8149 0.8443 1.0000  233 237 255  #e9edff

7700 K   2deg  0.3000 0.3105  5.402e+15    0.8244 0.8437 1.0000  234 237 255  #eaedff

7700 K  10deg  0.2982 0.3095  5.937e+15    0.8014 0.8363 1.0000  231 236 255  #e7ecff

7800 K   2deg  0.2990 0.3094  5.651e+15    0.8115 0.8351 1.0000  233 236 255  #e9ecff

7800 K  10deg  0.2971 0.3084  6.213e+15    0.7885 0.8285 1.0000  230 235 255  #e6ebff

7900 K   2deg  0.2980 0.3082  5.906e+15    0.7992 0.8268 1.0000  231 234 255  #e7eaff

7900 K  10deg  0.2961 0.3074  6.495e+15    0.7762 0.8211 1.0000  228 234 255  #e4eaff

8000 K   2deg  0.2970 0.3071  6.165e+15    0.7874 0.8187 1.0000  229 233 255  #e5e9ff

8000 K  10deg  0.2951 0.3064  6.783e+15    0.7644 0.8139 1.0000  227 233 255  #e3e9ff

8100 K   2deg  0.2961 0.3061  6.429e+15    0.7761 0.8110 1.0000  228 233 255  #e4e9ff

8100 K  10deg  0.2942 0.3054  7.076e+15    0.7531 0.8069 1.0000  225 232 255  #e1e8ff

8200 K   2deg  0.2952 0.3050  6.698e+15    0.7652 0.8035 1.0000  227 232 255  #e3e8ff

8200 K  10deg  0.2932 0.3044  7.376e+15    0.7423 0.8002 1.0000  224 231 255  #e0e7ff

8300 K   2deg  0.2943 0.3040  6.973e+15    0.7548 0.7963 1.0000  225 231 255  #e1e7ff

8300 K  10deg  0.2923 0.3035  7.681e+15    0.7319 0.7938 1.0000  222 230 255  #dee6ff

8400 K   2deg  0.2934 0.3030  7.252e+15    0.7449 0.7894 1.0000  224 230 255  #e0e6ff

8400 K  10deg  0.2914 0.3026  7.991e+15    0.7219 0.7875 1.0000  221 230 255  #dde6ff

8500 K   2deg  0.2926 0.3020  7.535e+15    0.7353 0.7827 1.0000  223 229 255  #dfe5ff

8500 K  10deg  0.2906 0.3017  8.307e+15    0.7123 0.7815 1.0000  220 229 255  #dce5ff

8600 K   2deg  0.2917 0.3011  7.824e+15    0.7260 0.7762 1.0000  221 228 255  #dde4ff

8600 K  10deg  0.2897 0.3008  8.628e+15    0.7030 0.7757 1.0000  218 228 255  #dae4ff

8700 K   2deg  0.2910 0.3001  8.117e+15    0.7172 0.7699 1.0000  220 227 255  #dce3ff

8700 K  10deg  0.2889 0.3000  8.954e+15    0.6941 0.7700 1.0000  217 227 255  #d9e3ff

8800 K   2deg  0.2902 0.2992  8.415e+15    0.7086 0.7638 1.0000  219 226 255  #dbe2ff

8800 K  10deg  0.2881 0.2992  9.285e+15    0.6856 0.7645 1.0000  216 227 255  #d8e3ff

8900 K   2deg  0.2894 0.2983  8.717e+15    0.7004 0.7579 1.0000  218 226 255  #dae2ff

8900 K  10deg  0.2874 0.2983  9.622e+15    0.6773 0.7593 1.0000  215 226 255  #d7e2ff

9000 K   2deg  0.2887 0.2975  9.023e+15    0.6925 0.7522 1.0000  217 225 255  #d9e1ff

9000 K  10deg  0.2866 0.2976  9.964e+15    0.6693 0.7541 1.0000  214 225 255  #d6e1ff

9100 K   2deg  0.2880 0.2966  9.334e+15    0.6848 0.7467 1.0000  216 224 255  #d8e0ff

9100 K  10deg  0.2859 0.2968  1.031e+16    0.6617 0.7492 1.0000  212 225 255  #d4e1ff

9200 K   2deg  0.2873 0.2958  9.649e+15    0.6774 0.7414 1.0000  215 223 255  #d7dfff

9200 K  10deg  0.2852 0.2960  1.066e+16    0.6543 0.7444 1.0000  211 224 255  #d3e0ff

9300 K   2deg  0.2866 0.2950  9.969e+15    0.6703 0.7362 1.0000  214 223 255  #d6dfff

9300 K  10deg  0.2845 0.2953  1.102e+16    0.6471 0.7397 1.0000  210 223 255  #d2dfff

9400 K   2deg  0.2860 0.2942  1.029e+16    0.6635 0.7311 1.0000  213 222 255  #d5deff

9400 K  10deg  0.2839 0.2946  1.138e+16    0.6402 0.7352 1.0000  209 223 255  #d1dfff

9500 K   2deg  0.2853 0.2934  1.062e+16    0.6568 0.7263 1.0000  212 221 255  #d4ddff

9500 K  10deg  0.2832 0.2939  1.174e+16    0.6335 0.7308 1.0000  208 222 255  #d0deff

9600 K   2deg  0.2847 0.2927  1.095e+16    0.6504 0.7215 1.0000  211 221 255  #d3ddff

9600 K  10deg  0.2826 0.2932  1.211e+16    0.6271 0.7265 1.0000  207 221 255  #cfddff

9700 K   2deg  0.2841 0.2919  1.129e+16    0.6442 0.7169 1.0000  210 220 255  #d2dcff

9700 K  10deg  0.2820 0.2925  1.249e+16    0.6208 0.7224 1.0000  207 221 255  #cfddff

9800 K   2deg  0.2835 0.2912  1.163e+16    0.6382 0.7124 1.0000  209 220 255  #d1dcff

9800 K  10deg  0.2813 0.2918  1.287e+16    0.6148 0.7183 1.0000  206 220 255  #cedcff

9900 K   2deg  0.2829 0.2905  1.197e+16    0.6324 0.7081 1.0000  208 219 255  #d0dbff

9900 K  10deg  0.2808 0.2912  1.325e+16    0.6089 0.7144 1.0000  205 220 255  #cddcff

10000 K   2deg  0.2824 0.2898  1.231e+16    0.6268 0.7039 1.0000  207 218 255  #cfdaff

10000 K  10deg  0.2802 0.2905  1.364e+16    0.6033 0.7106 1.0000  204 219 255  #ccdbff

10100 K   2deg  0.2818 0.2891  1.267e+16    0.6213 0.6998 1.0000  207 218 255  #cfdaff

10100 K  10deg  0.2796 0.2899  1.403e+16    0.5978 0.7069 1.0000  203 219 255  #cbdbff

10200 K   2deg  0.2813 0.2884  1.302e+16    0.6161 0.6958 1.0000  206 217 255  #ced9ff

10200 K  10deg  0.2791 0.2893  1.442e+16    0.5925 0.7033 1.0000  202 218 255  #cadaff

10300 K   2deg  0.2807 0.2878  1.338e+16    0.6109 0.6919 1.0000  205 217 255  #cdd9ff

10300 K  10deg  0.2785 0.2887  1.482e+16    0.5873 0.6998 1.0000  201 218 255  #c9daff

10400 K   2deg  0.2802 0.2871  1.374e+16    0.6060 0.6881 1.0000  204 216 255  #ccd8ff

10400 K  10deg  0.2780 0.2881  1.523e+16    0.5823 0.6964 1.0000  201 217 255  #c9d9ff

10500 K   2deg  0.2797 0.2865  1.410e+16    0.6012 0.6844 1.0000  204 216 255  #ccd8ff

10500 K  10deg  0.2775 0.2876  1.563e+16    0.5774 0.6930 1.0000  200 217 255  #c8d9ff

10600 K   2deg  0.2792 0.2859  1.447e+16    0.5965 0.6808 1.0000  203 215 255  #cbd7ff

10600 K  10deg  0.2770 0.2870  1.605e+16    0.5727 0.6898 1.0000  199 216 255  #c7d8ff

10700 K   2deg  0.2788 0.2853  1.484e+16    0.5919 0.6773 1.0000  202 215 255  #cad7ff

10700 K  10deg  0.2765 0.2865  1.646e+16    0.5681 0.6866 1.0000  199 216 255  #c7d8ff

10800 K   2deg  0.2783 0.2847  1.521e+16    0.5875 0.6739 1.0000  202 214 255  #cad6ff

10800 K  10deg  0.2761 0.2859  1.688e+16    0.5637 0.6836 1.0000  198 216 255  #c6d8ff

10900 K   2deg  0.2778 0.2841  1.559e+16    0.5833 0.6706 1.0000  201 214 255  #c9d6ff

10900 K  10deg  0.2756 0.2854  1.730e+16    0.5593 0.6806 1.0000  197 215 255  #c5d7ff

11000 K   2deg  0.2774 0.2836  1.597e+16    0.5791 0.6674 1.0000  200 213 255  #c8d5ff

11000 K  10deg  0.2751 0.2849  1.773e+16    0.5551 0.6776 1.0000  196 215 255  #c4d7ff

11100 K   2deg  0.2770 0.2830  1.636e+16    0.5750 0.6642 1.0000  200 213 255  #c8d5ff

11100 K  10deg  0.2747 0.2844  1.816e+16    0.5510 0.6748 1.0000  196 214 255  #c4d6ff

11200 K   2deg  0.2765 0.2825  1.674e+16    0.5711 0.6611 1.0000  199 212 255  #c7d4ff

11200 K  10deg  0.2743 0.2839  1.859e+16    0.5470 0.6720 1.0000  195 214 255  #c3d6ff

11300 K   2deg  0.2761 0.2819  1.713e+16    0.5673 0.6581 1.0000  198 212 255  #c6d4ff

11300 K  10deg  0.2738 0.2834  1.903e+16    0.5432 0.6693 1.0000  195 214 255  #c3d6ff

11400 K   2deg  0.2757 0.2814  1.752e+16    0.5636 0.6552 1.0000  198 212 255  #c6d4ff

11400 K  10deg  0.2734 0.2829  1.947e+16    0.5394 0.6666 1.0000  194 213 255  #c2d5ff

11500 K   2deg  0.2753 0.2809  1.792e+16    0.5599 0.6523 1.0000  197 211 255  #c5d3ff

11500 K  10deg  0.2730 0.2825  1.991e+16    0.5357 0.6640 1.0000  193 213 255  #c1d5ff

11600 K   2deg  0.2749 0.2804  1.832e+16    0.5564 0.6495 1.0000  197 211 255  #c5d3ff

11600 K  10deg  0.2726 0.2820  2.036e+16    0.5322 0.6615 1.0000  193 212 255  #c1d4ff

11700 K   2deg  0.2745 0.2799  1.872e+16    0.5530 0.6468 1.0000  196 210 255  #c4d2ff

11700 K  10deg  0.2722 0.2816  2.081e+16    0.5287 0.6590 1.0000  192 212 255  #c0d4ff

11800 K   2deg  0.2742 0.2794  1.912e+16    0.5496 0.6441 1.0000  196 210 255  #c4d2ff

11800 K  10deg  0.2719 0.2811  2.126e+16    0.5253 0.6566 1.0000  192 212 255  #c0d4ff

11900 K   2deg  0.2738 0.2789  1.953e+16    0.5463 0.6415 1.0000  195 210 255  #c3d2ff

11900 K  10deg  0.2715 0.2807  2.171e+16    0.5220 0.6542 1.0000  191 211 255  #bfd3ff

12000 K   2deg  0.2734 0.2785  1.994e+16    0.5431 0.6389 1.0000  195 209 255  #c3d1ff

12000 K  10deg  0.2711 0.2803  2.217e+16    0.5187 0.6519 1.0000  191 211 255  #bfd3ff

12100 K   2deg  0.2731 0.2780  2.035e+16    0.5400 0.6364 1.0000  194 209 255  #c2d1ff

12100 K  10deg  0.2708 0.2799  2.263e+16    0.5156 0.6497 1.0000  190 211 255  #bed3ff

12200 K   2deg  0.2727 0.2776  2.076e+16    0.5370 0.6340 1.0000  194 208 255  #c2d0ff

12200 K  10deg  0.2704 0.2794  2.310e+16    0.5125 0.6474 1.0000  190 210 255  #bed2ff

12300 K   2deg  0.2724 0.2771  2.118e+16    0.5340 0.6316 1.0000  193 208 255  #c1d0ff

12300 K  10deg  0.2701 0.2790  2.356e+16    0.5095 0.6453 1.0000  189 210 255  #bdd2ff

12400 K   2deg  0.2721 0.2767  2.160e+16    0.5312 0.6293 1.0000  193 208 255  #c1d0ff

12400 K  10deg  0.2697 0.2786  2.403e+16    0.5066 0.6432 1.0000  189 210 255  #bdd2ff

12500 K   2deg  0.2717 0.2763  2.202e+16    0.5283 0.6270 1.0000  192 207 255  #c0cfff

12500 K  10deg  0.2694 0.2783  2.451e+16    0.5037 0.6411 1.0000  188 210 255  #bcd2ff

12600 K   2deg  0.2714 0.2758  2.244e+16    0.5256 0.6247 1.0000  192 207 255  #c0cfff

12600 K  10deg  0.2691 0.2779  2.498e+16    0.5009 0.6391 1.0000  188 209 255  #bcd1ff

12700 K   2deg  0.2711 0.2754  2.287e+16    0.5229 0.6225 1.0000  191 207 255  #bfcfff

12700 K  10deg  0.2688 0.2775  2.546e+16    0.4982 0.6371 1.0000  187 209 255  #bbd1ff

12800 K   2deg  0.2708 0.2750  2.330e+16    0.5203 0.6204 1.0000  191 206 255  #bfceff

12800 K  10deg  0.2684 0.2771  2.594e+16    0.4955 0.6351 1.0000  187 209 255  #bbd1ff

12900 K   2deg  0.2705 0.2746  2.373e+16    0.5177 0.6183 1.0000  190 206 255  #beceff

12900 K  10deg  0.2681 0.2768  2.643e+16    0.4929 0.6332 1.0000  186 208 255  #bad0ff

13000 K   2deg  0.2702 0.2742  2.416e+16    0.5152 0.6162 1.0000  190 206 255  #beceff

13000 K  10deg  0.2678 0.2764  2.691e+16    0.4904 0.6314 1.0000  186 208 255  #bad0ff

13100 K   2deg  0.2699 0.2738  2.459e+16    0.5128 0.6142 1.0000  190 206 255  #beceff

13100 K  10deg  0.2675 0.2761  2.740e+16    0.4879 0.6295 1.0000  185 208 255  #b9d0ff

13200 K   2deg  0.2696 0.2735  2.503e+16    0.5104 0.6122 1.0000  189 205 255  #bdcdff

13200 K  10deg  0.2673 0.2757  2.789e+16    0.4854 0.6277 1.0000  185 208 255  #b9d0ff

13300 K   2deg  0.2694 0.2731  2.547e+16    0.5080 0.6103 1.0000  189 205 255  #bdcdff

13300 K  10deg  0.2670 0.2754  2.838e+16    0.4831 0.6260 1.0000  185 207 255  #b9cfff

13400 K   2deg  0.2691 0.2727  2.591e+16    0.5057 0.6084 1.0000  188 205 255  #bccdff

13400 K  10deg  0.2667 0.2751  2.888e+16    0.4807 0.6243 1.0000  184 207 255  #b8cfff

13500 K   2deg  0.2688 0.2724  2.635e+16    0.5035 0.6065 1.0000  188 204 255  #bcccff

13500 K  10deg  0.2664 0.2747  2.938e+16    0.4785 0.6226 1.0000  184 207 255  #b8cfff

13600 K   2deg  0.2686 0.2720  2.680e+16    0.5013 0.6047 1.0000  188 204 255  #bcccff

13600 K  10deg  0.2662 0.2744  2.988e+16    0.4762 0.6209 1.0000  183 207 255  #b7cfff

13700 K   2deg  0.2683 0.2717  2.724e+16    0.4991 0.6029 1.0000  187 204 255  #bbccff

13700 K  10deg  0.2659 0.2741  3.038e+16    0.4740 0.6193 1.0000  183 206 255  #b7ceff

13800 K   2deg  0.2680 0.2713  2.769e+16    0.4970 0.6012 1.0000  187 204 255  #bbccff

13800 K  10deg  0.2656 0.2738  3.089e+16    0.4719 0.6177 1.0000  183 206 255  #b7ceff

13900 K   2deg  0.2678 0.2710  2.814e+16    0.4950 0.5994 1.0000  187 203 255  #bbcbff

13900 K  10deg  0.2654 0.2735  3.139e+16    0.4698 0.6161 1.0000  182 206 255  #b6ceff

14000 K   2deg  0.2675 0.2707  2.860e+16    0.4930 0.5978 1.0000  186 203 255  #bacbff

14000 K  10deg  0.2651 0.2732  3.190e+16    0.4677 0.6146 1.0000  182 206 255  #b6ceff

14100 K   2deg  0.2673 0.2703  2.905e+16    0.4910 0.5961 1.0000  186 203 255  #bacbff

14100 K  10deg  0.2649 0.2729  3.241e+16    0.4657 0.6131 1.0000  182 205 255  #b6cdff

14200 K   2deg  0.2671 0.2700  2.951e+16    0.4891 0.5945 1.0000  186 203 255  #bacbff

14200 K  10deg  0.2646 0.2726  3.293e+16    0.4638 0.6116 1.0000  181 205 255  #b5cdff

14300 K   2deg  0.2668 0.2697  2.997e+16    0.4872 0.5929 1.0000  185 202 255  #b9caff

14300 K  10deg  0.2644 0.2723  3.344e+16    0.4618 0.6102 1.0000  181 205 255  #b5cdff

14400 K   2deg  0.2666 0.2694  3.043e+16    0.4853 0.5913 1.0000  185 202 255  #b9caff

14400 K  10deg  0.2642 0.2720  3.396e+16    0.4599 0.6087 1.0000  181 205 255  #b5cdff

14500 K   2deg  0.2664 0.2691  3.089e+16    0.4835 0.5898 1.0000  185 202 255  #b9caff

14500 K  10deg  0.2639 0.2718  3.448e+16    0.4581 0.6073 1.0000  180 205 255  #b4cdff

14600 K   2deg  0.2662 0.2688  3.135e+16    0.4817 0.5882 1.0000  184 202 255  #b8caff

14600 K  10deg  0.2637 0.2715  3.500e+16    0.4563 0.6060 1.0000  180 204 255  #b4ccff

14700 K   2deg  0.2659 0.2685  3.182e+16    0.4799 0.5868 1.0000  184 201 255  #b8c9ff

14700 K  10deg  0.2635 0.2712  3.552e+16    0.4545 0.6046 1.0000  180 204 255  #b4ccff

14800 K   2deg  0.2657 0.2682  3.228e+16    0.4782 0.5853 1.0000  184 201 255  #b8c9ff

14800 K  10deg  0.2633 0.2709  3.605e+16    0.4527 0.6033 1.0000  179 204 255  #b3ccff

14900 K   2deg  0.2655 0.2679  3.275e+16    0.4765 0.5839 1.0000  184 201 255  #b8c9ff

14900 K  10deg  0.2631 0.2707  3.657e+16    0.4510 0.6020 1.0000  179 204 255  #b3ccff

15000 K   2deg  0.2653 0.2676  3.322e+16    0.4749 0.5824 1.0000  183 201 255  #b7c9ff

15000 K  10deg  0.2629 0.2704  3.710e+16    0.4493 0.6007 1.0000  179 204 255  #b3ccff

15100 K   2deg  0.2651 0.2673  3.369e+16    0.4733 0.5811 1.0000  183 201 255  #b7c9ff

15100 K  10deg  0.2626 0.2702  3.763e+16    0.4477 0.5994 1.0000  178 203 255  #b2cbff

15200 K   2deg  0.2649 0.2671  3.416e+16    0.4717 0.5797 1.0000  183 200 255  #b7c8ff

15200 K  10deg  0.2624 0.2699  3.817e+16    0.4460 0.5982 1.0000  178 203 255  #b2cbff

15300 K   2deg  0.2647 0.2668  3.464e+16    0.4701 0.5784 1.0000  182 200 255  #b6c8ff

15300 K  10deg  0.2622 0.2697  3.870e+16    0.4445 0.5970 1.0000  178 203 255  #b2cbff

15400 K   2deg  0.2645 0.2665  3.511e+16    0.4686 0.5770 1.0000  182 200 255  #b6c8ff

15400 K  10deg  0.2620 0.2694  3.923e+16    0.4429 0.5958 1.0000  178 203 255  #b2cbff

15500 K   2deg  0.2643 0.2663  3.559e+16    0.4671 0.5757 1.0000  182 200 255  #b6c8ff

15500 K  10deg  0.2618 0.2692  3.977e+16    0.4413 0.5946 1.0000  177 203 255  #b1cbff

15600 K   2deg  0.2641 0.2660  3.607e+16    0.4656 0.5745 1.0000  182 200 255  #b6c8ff

15600 K  10deg  0.2617 0.2690  4.031e+16    0.4398 0.5935 1.0000  177 202 255  #b1caff

15700 K   2deg  0.2639 0.2657  3.655e+16    0.4641 0.5732 1.0000  181 199 255  #b5c7ff

15700 K  10deg  0.2615 0.2687  4.085e+16    0.4384 0.5923 1.0000  177 202 255  #b1caff

15800 K   2deg  0.2638 0.2655  3.703e+16    0.4627 0.5720 1.0000  181 199 255  #b5c7ff

15800 K  10deg  0.2613 0.2685  4.139e+16    0.4369 0.5912 1.0000  177 202 255  #b1caff

15900 K   2deg  0.2636 0.2652  3.751e+16    0.4613 0.5708 1.0000  181 199 255  #b5c7ff

15900 K  10deg  0.2611 0.2683  4.194e+16    0.4355 0.5901 1.0000  176 202 255  #b0caff

16000 K   2deg  0.2634 0.2650  3.800e+16    0.4599 0.5696 1.0000  181 199 255  #b5c7ff

16000 K  10deg  0.2609 0.2681  4.248e+16    0.4341 0.5890 1.0000  176 202 255  #b0caff

16100 K   2deg  0.2632 0.2648  3.848e+16    0.4586 0.5684 1.0000  180 199 255  #b4c7ff

16100 K  10deg  0.2607 0.2678  4.303e+16    0.4327 0.5879 1.0000  176 202 255  #b0caff

16200 K   2deg  0.2631 0.2645  3.897e+16    0.4572 0.5673 1.0000  180 198 255  #b4c6ff

16200 K  10deg  0.2606 0.2676  4.358e+16    0.4313 0.5869 1.0000  175 201 255  #afc9ff

16300 K   2deg  0.2629 0.2643  3.946e+16    0.4559 0.5661 1.0000  180 198 255  #b4c6ff

16300 K  10deg  0.2604 0.2674  4.413e+16    0.4300 0.5859 1.0000  175 201 255  #afc9ff

16400 K   2deg  0.2627 0.2641  3.995e+16    0.4546 0.5650 1.0000  180 198 255  #b4c6ff

16400 K  10deg  0.2602 0.2672  4.468e+16    0.4287 0.5848 1.0000  175 201 255  #afc9ff

16500 K   2deg  0.2626 0.2638  4.044e+16    0.4534 0.5639 1.0000  179 198 255  #b3c6ff

16500 K  10deg  0.2601 0.2670  4.523e+16    0.4274 0.5838 1.0000  175 201 255  #afc9ff

16600 K   2deg  0.2624 0.2636  4.093e+16    0.4521 0.5628 1.0000  179 198 255  #b3c6ff

16600 K  10deg  0.2599 0.2668  4.579e+16    0.4261 0.5829 1.0000  175 201 255  #afc9ff

16700 K   2deg  0.2622 0.2634  4.143e+16    0.4509 0.5617 1.0000  179 198 255  #b3c6ff

16700 K  10deg  0.2597 0.2666  4.634e+16    0.4249 0.5819 1.0000  174 201 255  #aec9ff

16800 K   2deg  0.2621 0.2632  4.192e+16    0.4497 0.5607 1.0000  179 197 255  #b3c5ff

16800 K  10deg  0.2596 0.2664  4.690e+16    0.4236 0.5809 1.0000  174 201 255  #aec9ff

16900 K   2deg  0.2619 0.2629  4.241e+16    0.4485 0.5597 1.0000  179 197 255  #b3c5ff

16900 K  10deg  0.2594 0.2662  4.746e+16    0.4224 0.5800 1.0000  174 200 255  #aec8ff

17000 K   2deg  0.2618 0.2627  4.291e+16    0.4474 0.5586 1.0000  178 197 255  #b2c5ff

17000 K  10deg  0.2593 0.2660  4.802e+16    0.4212 0.5791 1.0000  174 200 255  #aec8ff

17100 K   2deg  0.2616 0.2625  4.341e+16    0.4462 0.5576 1.0000  178 197 255  #b2c5ff

17100 K  10deg  0.2591 0.2658  4.858e+16    0.4201 0.5781 1.0000  173 200 255  #adc8ff

17200 K   2deg  0.2615 0.2623  4.391e+16    0.4451 0.5566 1.0000  178 197 255  #b2c5ff

17200 K  10deg  0.2590 0.2656  4.914e+16    0.4189 0.5772 1.0000  173 200 255  #adc8ff

17300 K   2deg  0.2613 0.2621  4.441e+16    0.4440 0.5557 1.0000  178 197 255  #b2c5ff

17300 K  10deg  0.2588 0.2654  4.971e+16    0.4178 0.5763 1.0000  173 200 255  #adc8ff

17400 K   2deg  0.2612 0.2619  4.491e+16    0.4429 0.5547 1.0000  178 196 255  #b2c4ff

17400 K  10deg  0.2587 0.2653  5.027e+16    0.4167 0.5755 1.0000  173 200 255  #adc8ff

17500 K   2deg  0.2610 0.2617  4.541e+16    0.4418 0.5538 1.0000  177 196 255  #b1c4ff

17500 K  10deg  0.2585 0.2651  5.084e+16    0.4156 0.5746 1.0000  173 200 255  #adc8ff

17600 K   2deg  0.2609 0.2615  4.592e+16    0.4408 0.5528 1.0000  177 196 255  #b1c4ff

17600 K  10deg  0.2584 0.2649  5.141e+16    0.4145 0.5738 1.0000  172 199 255  #acc7ff

17700 K   2deg  0.2608 0.2613  4.642e+16    0.4397 0.5519 1.0000  177 196 255  #b1c4ff

17700 K  10deg  0.2582 0.2647  5.197e+16    0.4134 0.5729 1.0000  172 199 255  #acc7ff

17800 K   2deg  0.2606 0.2611  4.693e+16    0.4387 0.5510 1.0000  177 196 255  #b1c4ff

17800 K  10deg  0.2581 0.2645  5.254e+16    0.4124 0.5721 1.0000  172 199 255  #acc7ff

17900 K   2deg  0.2605 0.2609  4.743e+16    0.4377 0.5501 1.0000  177 196 255  #b1c4ff

17900 K  10deg  0.2580 0.2644  5.312e+16    0.4113 0.5713 1.0000  172 199 255  #acc7ff

18000 K   2deg  0.2604 0.2607  4.794e+16    0.4367 0.5492 1.0000  176 196 255  #b0c4ff

18000 K  10deg  0.2578 0.2642  5.369e+16    0.4103 0.5705 1.0000  172 199 255  #acc7ff

18100 K   2deg  0.2602 0.2606  4.845e+16    0.4357 0.5483 1.0000  176 195 255  #b0c3ff

18100 K  10deg  0.2577 0.2640  5.426e+16    0.4093 0.5697 1.0000  171 199 255  #abc7ff

18200 K   2deg  0.2601 0.2604  4.896e+16    0.4348 0.5475 1.0000  176 195 255  #b0c3ff

18200 K  10deg  0.2575 0.2639  5.484e+16    0.4083 0.5689 1.0000  171 199 255  #abc7ff

18300 K   2deg  0.2600 0.2602  4.947e+16    0.4338 0.5466 1.0000  176 195 255  #b0c3ff

18300 K  10deg  0.2574 0.2637  5.541e+16    0.4074 0.5681 1.0000  171 199 255  #abc7ff

18400 K   2deg  0.2598 0.2600  4.998e+16    0.4329 0.5458 1.0000  176 195 255  #b0c3ff

18400 K  10deg  0.2573 0.2635  5.599e+16    0.4064 0.5674 1.0000  171 198 255  #abc6ff

18500 K   2deg  0.2597 0.2598  5.049e+16    0.4319 0.5450 1.0000  176 195 255  #b0c3ff

18500 K  10deg  0.2572 0.2634  5.657e+16    0.4055 0.5666 1.0000  171 198 255  #abc6ff

18600 K   2deg  0.2596 0.2597  5.100e+16    0.4310 0.5442 1.0000  175 195 255  #afc3ff

18600 K  10deg  0.2570 0.2632  5.715e+16    0.4045 0.5659 1.0000  170 198 255  #aac6ff

18700 K   2deg  0.2595 0.2595  5.152e+16    0.4301 0.5434 1.0000  175 195 255  #afc3ff

18700 K  10deg  0.2569 0.2631  5.773e+16    0.4036 0.5652 1.0000  170 198 255  #aac6ff

18800 K   2deg  0.2593 0.2593  5.203e+16    0.4293 0.5426 1.0000  175 194 255  #afc2ff

18800 K  10deg  0.2568 0.2629  5.831e+16    0.4027 0.5644 1.0000  170 198 255  #aac6ff

18900 K   2deg  0.2592 0.2592  5.255e+16    0.4284 0.5418 1.0000  175 194 255  #afc2ff

18900 K  10deg  0.2567 0.2628  5.889e+16    0.4018 0.5637 1.0000  170 198 255  #aac6ff

19000 K   2deg  0.2591 0.2590  5.307e+16    0.4275 0.5410 1.0000  175 194 255  #afc2ff

19000 K  10deg  0.2566 0.2626  5.947e+16    0.4009 0.5630 1.0000  170 198 255  #aac6ff

19100 K   2deg  0.2590 0.2588  5.358e+16    0.4267 0.5403 1.0000  175 194 255  #afc2ff

19100 K  10deg  0.2564 0.2625  6.006e+16    0.4001 0.5623 1.0000  170 198 255  #aac6ff

19200 K   2deg  0.2589 0.2587  5.410e+16    0.4258 0.5395 1.0000  174 194 255  #aec2ff

19200 K  10deg  0.2563 0.2623  6.064e+16    0.3992 0.5616 1.0000  169 198 255  #a9c6ff

19300 K   2deg  0.2588 0.2585  5.462e+16    0.4250 0.5388 1.0000  174 194 255  #aec2ff

19300 K  10deg  0.2562 0.2622  6.123e+16    0.3984 0.5610 1.0000  169 197 255  #a9c5ff

19400 K   2deg  0.2587 0.2584  5.514e+16    0.4242 0.5381 1.0000  174 194 255  #aec2ff

19400 K  10deg  0.2561 0.2620  6.181e+16    0.3975 0.5603 1.0000  169 197 255  #a9c5ff

19500 K   2deg  0.2586 0.2582  5.566e+16    0.4234 0.5373 1.0000  174 194 255  #aec2ff

19500 K  10deg  0.2560 0.2619  6.240e+16    0.3967 0.5596 1.0000  169 197 255  #a9c5ff

19600 K   2deg  0.2584 0.2580  5.618e+16    0.4226 0.5366 1.0000  174 194 255  #aec2ff

19600 K  10deg  0.2559 0.2618  6.299e+16    0.3959 0.5590 1.0000  169 197 255  #a9c5ff

19700 K   2deg  0.2583 0.2579  5.671e+16    0.4218 0.5359 1.0000  174 193 255  #aec1ff

19700 K  10deg  0.2558 0.2616  6.358e+16    0.3951 0.5584 1.0000  169 197 255  #a9c5ff

19800 K   2deg  0.2582 0.2577  5.723e+16    0.4211 0.5352 1.0000  174 193 255  #aec1ff

19800 K  10deg  0.2557 0.2615  6.417e+16    0.3943 0.5577 1.0000  169 197 255  #a9c5ff

19900 K   2deg  0.2581 0.2576  5.775e+16    0.4203 0.5345 1.0000  173 193 255  #adc1ff

19900 K  10deg  0.2555 0.2613  6.476e+16    0.3935 0.5571 1.0000  168 197 255  #a8c5ff

20000 K   2deg  0.2580 0.2574  5.828e+16    0.4196 0.5339 1.0000  173 193 255  #adc1ff

20000 K  10deg  0.2554 0.2612  6.535e+16    0.3928 0.5565 1.0000  168 197 255  #a8c5ff

20100 K   2deg  0.2579 0.2573  5.880e+16    0.4188 0.5332 1.0000  173 193 255  #adc1ff

20100 K  10deg  0.2553 0.2611  6.595e+16    0.3920 0.5559 1.0000  168 197 255  #a8c5ff

20200 K   2deg  0.2578 0.2572  5.933e+16    0.4181 0.5325 1.0000  173 193 255  #adc1ff

20200 K  10deg  0.2552 0.2609  6.654e+16    0.3913 0.5553 1.0000  168 197 255  #a8c5ff

20300 K   2deg  0.2577 0.2570  5.986e+16    0.4174 0.5319 1.0000  173 193 255  #adc1ff

20300 K  10deg  0.2551 0.2608  6.714e+16    0.3905 0.5547 1.0000  168 196 255  #a8c4ff

20400 K   2deg  0.2576 0.2569  6.038e+16    0.4167 0.5312 1.0000  173 193 255  #adc1ff

20400 K  10deg  0.2550 0.2607  6.773e+16    0.3898 0.5541 1.0000  168 196 255  #a8c4ff

20500 K   2deg  0.2575 0.2567  6.091e+16    0.4160 0.5306 1.0000  173 193 255  #adc1ff

20500 K  10deg  0.2549 0.2606  6.833e+16    0.3891 0.5535 1.0000  168 196 255  #a8c4ff

20600 K   2deg  0.2574 0.2566  6.144e+16    0.4153 0.5300 1.0000  173 192 255  #adc0ff

20600 K  10deg  0.2548 0.2604  6.893e+16    0.3884 0.5529 1.0000  167 196 255  #a7c4ff

20700 K   2deg  0.2573 0.2565  6.197e+16    0.4146 0.5293 1.0000  172 192 255  #acc0ff

20700 K  10deg  0.2547 0.2603  6.952e+16    0.3877 0.5524 1.0000  167 196 255  #a7c4ff

20800 K   2deg  0.2572 0.2563  6.250e+16    0.4139 0.5287 1.0000  172 192 255  #acc0ff

20800 K  10deg  0.2546 0.2602  7.012e+16    0.3870 0.5518 1.0000  167 196 255  #a7c4ff

20900 K   2deg  0.2571 0.2562  6.303e+16    0.4133 0.5281 1.0000  172 192 255  #acc0ff

20900 K  10deg  0.2546 0.2601  7.072e+16    0.3863 0.5513 1.0000  167 196 255  #a7c4ff

21000 K   2deg  0.2571 0.2561  6.357e+16    0.4126 0.5275 1.0000  172 192 255  #acc0ff

21000 K  10deg  0.2545 0.2600  7.132e+16    0.3856 0.5507 1.0000  167 196 255  #a7c4ff

21100 K   2deg  0.2570 0.2559  6.410e+16    0.4119 0.5269 1.0000  172 192 255  #acc0ff

21100 K  10deg  0.2544 0.2598  7.193e+16    0.3850 0.5502 1.0000  167 196 255  #a7c4ff

21200 K   2deg  0.2569 0.2558  6.463e+16    0.4113 0.5264 1.0000  172 192 255  #acc0ff

21200 K  10deg  0.2543 0.2597  7.253e+16    0.3843 0.5496 1.0000  167 196 255  #a7c4ff

21300 K   2deg  0.2568 0.2557  6.517e+16    0.4107 0.5258 1.0000  172 192 255  #acc0ff

21300 K  10deg  0.2542 0.2596  7.313e+16    0.3836 0.5491 1.0000  166 196 255  #a6c4ff

21400 K   2deg  0.2567 0.2555  6.570e+16    0.4100 0.5252 1.0000  172 192 255  #acc0ff

21400 K  10deg  0.2541 0.2595  7.373e+16    0.3830 0.5486 1.0000  166 195 255  #a6c3ff

21500 K   2deg  0.2566 0.2554  6.623e+16    0.4094 0.5246 1.0000  171 192 255  #abc0ff

21500 K  10deg  0.2540 0.2594  7.434e+16    0.3824 0.5481 1.0000  166 195 255  #a6c3ff

21600 K   2deg  0.2565 0.2553  6.677e+16    0.4088 0.5241 1.0000  171 192 255  #abc0ff

21600 K  10deg  0.2539 0.2593  7.494e+16    0.3817 0.5476 1.0000  166 195 255  #a6c3ff

21700 K   2deg  0.2564 0.2552  6.731e+16    0.4082 0.5235 1.0000  171 191 255  #abbfff

21700 K  10deg  0.2538 0.2591  7.555e+16    0.3811 0.5471 1.0000  166 195 255  #a6c3ff

21800 K   2deg  0.2564 0.2550  6.784e+16    0.4076 0.5230 1.0000  171 191 255  #abbfff

21800 K  10deg  0.2537 0.2590  7.616e+16    0.3805 0.5466 1.0000  166 195 255  #a6c3ff

21900 K   2deg  0.2563 0.2549  6.838e+16    0.4070 0.5224 1.0000  171 191 255  #abbfff

21900 K  10deg  0.2537 0.2589  7.676e+16    0.3799 0.5461 1.0000  166 195 255  #a6c3ff

22000 K   2deg  0.2562 0.2548  6.892e+16    0.4064 0.5219 1.0000  171 191 255  #abbfff

22000 K  10deg  0.2536 0.2588  7.737e+16    0.3793 0.5456 1.0000  166 195 255  #a6c3ff

22100 K   2deg  0.2561 0.2547  6.946e+16    0.4059 0.5214 1.0000  171 191 255  #abbfff

22100 K  10deg  0.2535 0.2587  7.798e+16    0.3787 0.5451 1.0000  165 195 255  #a5c3ff

22200 K   2deg  0.2560 0.2546  7.000e+16    0.4053 0.5209 1.0000  171 191 255  #abbfff

22200 K  10deg  0.2534 0.2586  7.859e+16    0.3781 0.5446 1.0000  165 195 255  #a5c3ff

22300 K   2deg  0.2559 0.2545  7.054e+16    0.4047 0.5203 1.0000  171 191 255  #abbfff

22300 K  10deg  0.2533 0.2585  7.920e+16    0.3776 0.5441 1.0000  165 195 255  #a5c3ff

22400 K   2deg  0.2559 0.2543  7.108e+16    0.4042 0.5198 1.0000  170 191 255  #aabfff

22400 K  10deg  0.2533 0.2584  7.981e+16    0.3770 0.5437 1.0000  165 195 255  #a5c3ff

22500 K   2deg  0.2558 0.2542  7.162e+16    0.4036 0.5193 1.0000  170 191 255  #aabfff

22500 K  10deg  0.2532 0.2583  8.042e+16    0.3764 0.5432 1.0000  165 195 255  #a5c3ff

22600 K   2deg  0.2557 0.2541  7.216e+16    0.4031 0.5188 1.0000  170 191 255  #aabfff

22600 K  10deg  0.2531 0.2582  8.103e+16    0.3759 0.5428 1.0000  165 195 255  #a5c3ff

22700 K   2deg  0.2556 0.2540  7.270e+16    0.4026 0.5183 1.0000  170 191 255  #aabfff

22700 K  10deg  0.2530 0.2581  8.164e+16    0.3753 0.5423 1.0000  165 194 255  #a5c2ff

22800 K   2deg  0.2556 0.2539  7.325e+16    0.4020 0.5178 1.0000  170 190 255  #aabeff

22800 K  10deg  0.2529 0.2580  8.226e+16    0.3748 0.5419 1.0000  165 194 255  #a5c2ff

22900 K   2deg  0.2555 0.2538  7.379e+16    0.4015 0.5174 1.0000  170 190 255  #aabeff

22900 K  10deg  0.2529 0.2579  8.287e+16    0.3742 0.5414 1.0000  165 194 255  #a5c2ff

23000 K   2deg  0.2554 0.2537  7.433e+16    0.4010 0.5169 1.0000  170 190 255  #aabeff

23000 K  10deg  0.2528 0.2578  8.349e+16    0.3737 0.5410 1.0000  164 194 255  #a4c2ff

23100 K   2deg  0.2553 0.2536  7.488e+16    0.4005 0.5164 1.0000  170 190 255  #aabeff

23100 K  10deg  0.2527 0.2577  8.410e+16    0.3732 0.5405 1.0000  164 194 255  #a4c2ff

23200 K   2deg  0.2553 0.2535  7.542e+16    0.4000 0.5159 1.0000  170 190 255  #aabeff

23200 K  10deg  0.2526 0.2576  8.472e+16    0.3726 0.5401 1.0000  164 194 255  #a4c2ff

23300 K   2deg  0.2552 0.2534  7.597e+16    0.3995 0.5155 1.0000  170 190 255  #aabeff

23300 K  10deg  0.2526 0.2575  8.533e+16    0.3721 0.5397 1.0000  164 194 255  #a4c2ff

23400 K   2deg  0.2551 0.2533  7.651e+16    0.3990 0.5150 1.0000  169 190 255  #a9beff

23400 K  10deg  0.2525 0.2574  8.595e+16    0.3716 0.5393 1.0000  164 194 255  #a4c2ff

23500 K   2deg  0.2551 0.2532  7.706e+16    0.3985 0.5146 1.0000  169 190 255  #a9beff

23500 K  10deg  0.2524 0.2573  8.657e+16    0.3711 0.5389 1.0000  164 194 255  #a4c2ff

23600 K   2deg  0.2550 0.2531  7.760e+16    0.3980 0.5141 1.0000  169 190 255  #a9beff

23600 K  10deg  0.2524 0.2572  8.718e+16    0.3706 0.5384 1.0000  164 194 255  #a4c2ff

23700 K   2deg  0.2549 0.2530  7.815e+16    0.3975 0.5137 1.0000  169 190 255  #a9beff

23700 K  10deg  0.2523 0.2572  8.780e+16    0.3701 0.5380 1.0000  164 194 255  #a4c2ff

23800 K   2deg  0.2548 0.2529  7.870e+16    0.3970 0.5132 1.0000  169 190 255  #a9beff

23800 K  10deg  0.2522 0.2571  8.842e+16    0.3696 0.5376 1.0000  164 194 255  #a4c2ff

23900 K   2deg  0.2548 0.2528  7.925e+16    0.3966 0.5128 1.0000  169 190 255  #a9beff

23900 K  10deg  0.2521 0.2570  8.904e+16    0.3692 0.5372 1.0000  164 194 255  #a4c2ff

24000 K   2deg  0.2547 0.2527  7.980e+16    0.3961 0.5123 1.0000  169 190 255  #a9beff

24000 K  10deg  0.2521 0.2569  8.966e+16    0.3687 0.5368 1.0000  163 194 255  #a3c2ff

24100 K   2deg  0.2546 0.2526  8.035e+16    0.3956 0.5119 1.0000  169 190 255  #a9beff

24100 K  10deg  0.2520 0.2568  9.028e+16    0.3682 0.5365 1.0000  163 194 255  #a3c2ff

24200 K   2deg  0.2546 0.2525  8.089e+16    0.3952 0.5115 1.0000  169 189 255  #a9bdff

24200 K  10deg  0.2519 0.2567  9.090e+16    0.3677 0.5361 1.0000  163 193 255  #a3c1ff

24300 K   2deg  0.2545 0.2524  8.144e+16    0.3947 0.5111 1.0000  169 189 255  #a9bdff

24300 K  10deg  0.2519 0.2566  9.152e+16    0.3673 0.5357 1.0000  163 193 255  #a3c1ff

24400 K   2deg  0.2544 0.2523  8.199e+16    0.3943 0.5107 1.0000  169 189 255  #a9bdff

24400 K  10deg  0.2518 0.2565  9.215e+16    0.3668 0.5353 1.0000  163 193 255  #a3c1ff

24500 K   2deg  0.2544 0.2522  8.255e+16    0.3938 0.5103 1.0000  168 189 255  #a8bdff

24500 K  10deg  0.2517 0.2565  9.277e+16    0.3664 0.5349 1.0000  163 193 255  #a3c1ff

24600 K   2deg  0.2543 0.2521  8.310e+16    0.3934 0.5098 1.0000  168 189 255  #a8bdff

24600 K  10deg  0.2517 0.2564  9.339e+16    0.3659 0.5346 1.0000  163 193 255  #a3c1ff

24700 K   2deg  0.2543 0.2520  8.365e+16    0.3930 0.5094 1.0000  168 189 255  #a8bdff

24700 K  10deg  0.2516 0.2563  9.401e+16    0.3655 0.5342 1.0000  163 193 255  #a3c1ff

24800 K   2deg  0.2542 0.2519  8.420e+16    0.3925 0.5090 1.0000  168 189 255  #a8bdff

24800 K  10deg  0.2516 0.2562  9.464e+16    0.3650 0.5338 1.0000  163 193 255  #a3c1ff

24900 K   2deg  0.2541 0.2518  8.475e+16    0.3921 0.5086 1.0000  168 189 255  #a8bdff

24900 K  10deg  0.2515 0.2561  9.526e+16    0.3646 0.5335 1.0000  163 193 255  #a3c1ff

25000 K   2deg  0.2541 0.2517  8.531e+16    0.3917 0.5083 1.0000  168 189 255  #a8bdff

25000 K  10deg  0.2514 0.2560  9.589e+16    0.3642 0.5331 1.0000  163 193 255  #a3c1ff

25100 K   2deg  0.2540 0.2516  8.586e+16    0.3913 0.5079 1.0000  168 189 255  #a8bdff

25100 K  10deg  0.2514 0.2560  9.651e+16    0.3637 0.5328 1.0000  162 193 255  #a2c1ff

25200 K   2deg  0.2540 0.2516  8.641e+16    0.3909 0.5075 1.0000  168 189 255  #a8bdff

25200 K  10deg  0.2513 0.2559  9.714e+16    0.3633 0.5324 1.0000  162 193 255  #a2c1ff

25300 K   2deg  0.2539 0.2515  8.697e+16    0.3905 0.5071 1.0000  168 189 255  #a8bdff

25300 K  10deg  0.2512 0.2558  9.777e+16    0.3629 0.5321 1.0000  162 193 255  #a2c1ff

25400 K   2deg  0.2538 0.2514  8.752e+16    0.3901 0.5067 1.0000  168 189 255  #a8bdff

25400 K  10deg  0.2512 0.2557  9.839e+16    0.3625 0.5317 1.0000  162 193 255  #a2c1ff

25500 K   2deg  0.2538 0.2513  8.808e+16    0.3897 0.5064 1.0000  168 189 255  #a8bdff

25500 K  10deg  0.2511 0.2557  9.902e+16    0.3621 0.5314 1.0000  162 193 255  #a2c1ff

25600 K   2deg  0.2537 0.2512  8.863e+16    0.3893 0.5060 1.0000  168 189 255  #a8bdff

25600 K  10deg  0.2511 0.2556  9.965e+16    0.3617 0.5310 1.0000  162 193 255  #a2c1ff

25700 K   2deg  0.2537 0.2511  8.919e+16    0.3889 0.5056 1.0000  167 188 255  #a7bcff

25700 K  10deg  0.2510 0.2555  1.003e+17    0.3613 0.5307 1.0000  162 193 255  #a2c1ff

25800 K   2deg  0.2536 0.2511  8.974e+16    0.3885 0.5053 1.0000  167 188 255  #a7bcff

25800 K  10deg  0.2510 0.2554  1.009e+17    0.3609 0.5304 1.0000  162 193 255  #a2c1ff

25900 K   2deg  0.2535 0.2510  9.030e+16    0.3881 0.5049 1.0000  167 188 255  #a7bcff

25900 K  10deg  0.2509 0.2554  1.015e+17    0.3605 0.5300 1.0000  162 192 255  #a2c0ff

26000 K   2deg  0.2535 0.2509  9.086e+16    0.3877 0.5045 1.0000  167 188 255  #a7bcff

26000 K  10deg  0.2508 0.2553  1.022e+17    0.3601 0.5297 1.0000  162 192 255  #a2c0ff

26100 K   2deg  0.2534 0.2508  9.141e+16    0.3874 0.5042 1.0000  167 188 255  #a7bcff

26100 K  10deg  0.2508 0.2552  1.028e+17    0.3597 0.5294 1.0000  162 192 255  #a2c0ff

26200 K   2deg  0.2534 0.2507  9.197e+16    0.3870 0.5038 1.0000  167 188 255  #a7bcff

26200 K  10deg  0.2507 0.2551  1.034e+17    0.3593 0.5291 1.0000  162 192 255  #a2c0ff

26300 K   2deg  0.2533 0.2507  9.253e+16    0.3866 0.5035 1.0000  167 188 255  #a7bcff

26300 K  10deg  0.2507 0.2551  1.041e+17    0.3589 0.5288 1.0000  162 192 255  #a2c0ff

26400 K   2deg  0.2533 0.2506  9.309e+16    0.3863 0.5032 1.0000  167 188 255  #a7bcff

26400 K  10deg  0.2506 0.2550  1.047e+17    0.3586 0.5284 1.0000  161 192 255  #a1c0ff

26500 K   2deg  0.2532 0.2505  9.365e+16    0.3859 0.5028 1.0000  167 188 255  #a7bcff

26500 K  10deg  0.2506 0.2549  1.053e+17    0.3582 0.5281 1.0000  161 192 255  #a1c0ff

26600 K   2deg  0.2532 0.2504  9.421e+16    0.3855 0.5025 1.0000  167 188 255  #a7bcff

26600 K  10deg  0.2505 0.2548  1.060e+17    0.3578 0.5278 1.0000  161 192 255  #a1c0ff

26700 K   2deg  0.2531 0.2503  9.476e+16    0.3852 0.5021 1.0000  167 188 255  #a7bcff

26700 K  10deg  0.2505 0.2548  1.066e+17    0.3575 0.5275 1.0000  161 192 255  #a1c0ff

26800 K   2deg  0.2531 0.2503  9.532e+16    0.3848 0.5018 1.0000  167 188 255  #a7bcff

26800 K  10deg  0.2504 0.2547  1.072e+17    0.3571 0.5272 1.0000  161 192 255  #a1c0ff

26900 K   2deg  0.2530 0.2502  9.588e+16    0.3845 0.5015 1.0000  167 188 255  #a7bcff

26900 K  10deg  0.2503 0.2546  1.078e+17    0.3567 0.5269 1.0000  161 192 255  #a1c0ff

27000 K   2deg  0.2530 0.2501  9.644e+16    0.3841 0.5012 1.0000  167 188 255  #a7bcff

27000 K  10deg  0.2503 0.2546  1.085e+17    0.3564 0.5266 1.0000  161 192 255  #a1c0ff

27100 K   2deg  0.2529 0.2500  9.701e+16    0.3838 0.5008 1.0000  166 188 255  #a6bcff

27100 K  10deg  0.2502 0.2545  1.091e+17    0.3560 0.5263 1.0000  161 192 255  #a1c0ff

27200 K   2deg  0.2529 0.2500  9.757e+16    0.3835 0.5005 1.0000  166 188 255  #a6bcff

27200 K  10deg  0.2502 0.2544  1.098e+17    0.3557 0.5260 1.0000  161 192 255  #a1c0ff

27300 K   2deg  0.2528 0.2499  9.813e+16    0.3831 0.5002 1.0000  166 188 255  #a6bcff

27300 K  10deg  0.2501 0.2544  1.104e+17    0.3553 0.5257 1.0000  161 192 255  #a1c0ff

27400 K   2deg  0.2528 0.2498  9.869e+16    0.3828 0.4999 1.0000  166 187 255  #a6bbff

27400 K  10deg  0.2501 0.2543  1.110e+17    0.3550 0.5255 1.0000  161 192 255  #a1c0ff

27500 K   2deg  0.2527 0.2497  9.925e+16    0.3825 0.4996 1.0000  166 187 255  #a6bbff

27500 K  10deg  0.2500 0.2542  1.117e+17    0.3546 0.5252 1.0000  161 192 255  #a1c0ff

27600 K   2deg  0.2527 0.2497  9.981e+16    0.3821 0.4993 1.0000  166 187 255  #a6bbff

27600 K  10deg  0.2500 0.2542  1.123e+17    0.3543 0.5249 1.0000  161 192 255  #a1c0ff

27700 K   2deg  0.2526 0.2496  1.004e+17    0.3818 0.4990 1.0000  166 187 255  #a6bbff

27700 K  10deg  0.2499 0.2541  1.129e+17    0.3540 0.5246 1.0000  161 192 255  #a1c0ff

27800 K   2deg  0.2526 0.2495  1.009e+17    0.3815 0.4987 1.0000  166 187 255  #a6bbff

27800 K  10deg  0.2499 0.2541  1.136e+17    0.3536 0.5243 1.0000  160 192 255  #a0c0ff

27900 K   2deg  0.2525 0.2495  1.015e+17    0.3812 0.4984 1.0000  166 187 255  #a6bbff

27900 K  10deg  0.2499 0.2540  1.142e+17    0.3533 0.5241 1.0000  160 192 255  #a0c0ff

28000 K   2deg  0.2525 0.2494  1.021e+17    0.3809 0.4981 1.0000  166 187 255  #a6bbff

28000 K  10deg  0.2498 0.2539  1.148e+17    0.3530 0.5238 1.0000  160 191 255  #a0bfff

28100 K   2deg  0.2524 0.2493  1.026e+17    0.3805 0.4978 1.0000  166 187 255  #a6bbff

28100 K  10deg  0.2498 0.2539  1.155e+17    0.3527 0.5235 1.0000  160 191 255  #a0bfff

28200 K   2deg  0.2524 0.2493  1.032e+17    0.3802 0.4975 1.0000  166 187 255  #a6bbff

28200 K  10deg  0.2497 0.2538  1.161e+17    0.3524 0.5232 1.0000  160 191 255  #a0bfff

28300 K   2deg  0.2523 0.2492  1.038e+17    0.3799 0.4972 1.0000  166 187 255  #a6bbff

28300 K  10deg  0.2497 0.2537  1.168e+17    0.3520 0.5230 1.0000  160 191 255  #a0bfff

28400 K   2deg  0.2523 0.2491  1.043e+17    0.3796 0.4969 1.0000  166 187 255  #a6bbff

28400 K  10deg  0.2496 0.2537  1.174e+17    0.3517 0.5227 1.0000  160 191 255  #a0bfff

28500 K   2deg  0.2523 0.2491  1.049e+17    0.3793 0.4966 1.0000  166 187 255  #a6bbff

28500 K  10deg  0.2496 0.2536  1.180e+17    0.3514 0.5225 1.0000  160 191 255  #a0bfff

28600 K   2deg  0.2522 0.2490  1.055e+17    0.3790 0.4963 1.0000  166 187 255  #a6bbff

28600 K  10deg  0.2495 0.2536  1.187e+17    0.3511 0.5222 1.0000  160 191 255  #a0bfff

28700 K   2deg  0.2522 0.2489  1.060e+17    0.3787 0.4960 1.0000  165 187 255  #a5bbff

28700 K  10deg  0.2495 0.2535  1.193e+17    0.3508 0.5219 1.0000  160 191 255  #a0bfff

28800 K   2deg  0.2521 0.2489  1.066e+17    0.3784 0.4958 1.0000  165 187 255  #a5bbff

28800 K  10deg  0.2494 0.2534  1.199e+17    0.3505 0.5217 1.0000  160 191 255  #a0bfff

28900 K   2deg  0.2521 0.2488  1.072e+17    0.3781 0.4955 1.0000  165 187 255  #a5bbff

28900 K  10deg  0.2494 0.2534  1.206e+17    0.3502 0.5214 1.0000  160 191 255  #a0bfff

29000 K   2deg  0.2520 0.2487  1.077e+17    0.3779 0.4952 1.0000  165 187 255  #a5bbff

29000 K  10deg  0.2493 0.2533  1.212e+17    0.3499 0.5212 1.0000  160 191 255  #a0bfff

29100 K   2deg  0.2520 0.2487  1.083e+17    0.3776 0.4949 1.0000  165 187 255  #a5bbff

29100 K  10deg  0.2493 0.2533  1.219e+17    0.3496 0.5209 1.0000  160 191 255  #a0bfff

29200 K   2deg  0.2519 0.2486  1.089e+17    0.3773 0.4947 1.0000  165 187 255  #a5bbff

29200 K  10deg  0.2493 0.2532  1.225e+17    0.3493 0.5207 1.0000  160 191 255  #a0bfff

29300 K   2deg  0.2519 0.2485  1.094e+17    0.3770 0.4944 1.0000  165 187 255  #a5bbff

29300 K  10deg  0.2492 0.2532  1.232e+17    0.3490 0.5204 1.0000  159 191 255  #9fbfff

29400 K   2deg  0.2519 0.2485  1.100e+17    0.3767 0.4941 1.0000  165 187 255  #a5bbff

29400 K  10deg  0.2492 0.2531  1.238e+17    0.3487 0.5202 1.0000  159 191 255  #9fbfff

29500 K   2deg  0.2518 0.2484  1.106e+17    0.3764 0.4939 1.0000  165 186 255  #a5baff

29500 K  10deg  0.2491 0.2530  1.244e+17    0.3485 0.5200 1.0000  159 191 255  #9fbfff

29600 K   2deg  0.2518 0.2484  1.111e+17    0.3762 0.4936 1.0000  165 186 255  #a5baff

29600 K  10deg  0.2491 0.2530  1.251e+17    0.3482 0.5197 1.0000  159 191 255  #9fbfff

29700 K   2deg  0.2517 0.2483  1.117e+17    0.3759 0.4934 1.0000  165 186 255  #a5baff

29700 K  10deg  0.2490 0.2529  1.257e+17    0.3479 0.5195 1.0000  159 191 255  #9fbfff

29800 K   2deg  0.2517 0.2482  1.123e+17    0.3756 0.4931 1.0000  165 186 255  #a5baff

29800 K  10deg  0.2490 0.2529  1.264e+17    0.3476 0.5192 1.0000  159 191 255  #9fbfff

29900 K   2deg  0.2517 0.2482  1.128e+17    0.3754 0.4928 1.0000  165 186 255  #a5baff

29900 K  10deg  0.2490 0.2528  1.270e+17    0.3473 0.5190 1.0000  159 191 255  #9fbfff

30000 K   2deg  0.2516 0.2481  1.134e+17    0.3751 0.4926 1.0000  165 186 255  #a5baff

30000 K  10deg  0.2489 0.2528  1.277e+17    0.3471 0.5188 1.0000  159 191 255  #9fbfff

30100 K   2deg  0.2516 0.2481  1.140e+17    0.3748 0.4923 1.0000  165 186 255  #a5baff

30100 K  10deg  0.2489 0.2527  1.283e+17    0.3468 0.5186 1.0000  159 191 255  #9fbfff

30200 K   2deg  0.2515 0.2480  1.145e+17    0.3746 0.4921 1.0000  165 186 255  #a5baff

30200 K  10deg  0.2488 0.2527  1.289e+17    0.3465 0.5183 1.0000  159 191 255  #9fbfff

30300 K   2deg  0.2515 0.2480  1.151e+17    0.3743 0.4918 1.0000  165 186 255  #a5baff

30300 K  10deg  0.2488 0.2526  1.296e+17    0.3463 0.5181 1.0000  159 191 255  #9fbfff

30400 K   2deg  0.2515 0.2479  1.157e+17    0.3741 0.4916 1.0000  165 186 255  #a5baff

30400 K  10deg  0.2488 0.2526  1.302e+17    0.3460 0.5179 1.0000  159 190 255  #9fbeff

30500 K   2deg  0.2514 0.2478  1.162e+17    0.3738 0.4914 1.0000  165 186 255  #a5baff

30500 K  10deg  0.2487 0.2525  1.309e+17    0.3457 0.5177 1.0000  159 190 255  #9fbeff

30600 K   2deg  0.2514 0.2478  1.168e+17    0.3735 0.4911 1.0000  164 186 255  #a4baff

30600 K  10deg  0.2487 0.2525  1.315e+17    0.3455 0.5174 1.0000  159 190 255  #9fbeff

30700 K   2deg  0.2513 0.2477  1.174e+17    0.3733 0.4909 1.0000  164 186 255  #a4baff

30700 K  10deg  0.2487 0.2524  1.322e+17    0.3452 0.5172 1.0000  159 190 255  #9fbeff

30800 K   2deg  0.2513 0.2477  1.180e+17    0.3730 0.4906 1.0000  164 186 255  #a4baff

30800 K  10deg  0.2486 0.2524  1.328e+17    0.3450 0.5170 1.0000  159 190 255  #9fbeff

30900 K   2deg  0.2513 0.2476  1.185e+17    0.3728 0.4904 1.0000  164 186 255  #a4baff

30900 K  10deg  0.2486 0.2523  1.335e+17    0.3447 0.5168 1.0000  159 190 255  #9fbeff

31000 K   2deg  0.2512 0.2476  1.191e+17    0.3726 0.4902 1.0000  164 186 255  #a4baff

31000 K  10deg  0.2485 0.2523  1.341e+17    0.3444 0.5166 1.0000  159 190 255  #9fbeff

31100 K   2deg  0.2512 0.2475  1.197e+17    0.3723 0.4899 1.0000  164 186 255  #a4baff

31100 K  10deg  0.2485 0.2522  1.348e+17    0.3442 0.5164 1.0000  158 190 255  #9ebeff

31200 K   2deg  0.2512 0.2474  1.202e+17    0.3721 0.4897 1.0000  164 186 255  #a4baff

31200 K  10deg  0.2485 0.2522  1.354e+17    0.3439 0.5161 1.0000  158 190 255  #9ebeff

31300 K   2deg  0.2511 0.2474  1.208e+17    0.3718 0.4895 1.0000  164 186 255  #a4baff

31300 K  10deg  0.2484 0.2521  1.360e+17    0.3437 0.5159 1.0000  158 190 255  #9ebeff

31400 K   2deg  0.2511 0.2473  1.214e+17    0.3716 0.4893 1.0000  164 186 255  #a4baff

31400 K  10deg  0.2484 0.2521  1.367e+17    0.3435 0.5157 1.0000  158 190 255  #9ebeff

31500 K   2deg  0.2511 0.2473  1.220e+17    0.3714 0.4890 1.0000  164 186 255  #a4baff

31500 K  10deg  0.2484 0.2520  1.373e+17    0.3432 0.5155 1.0000  158 190 255  #9ebeff

31600 K   2deg  0.2510 0.2472  1.225e+17    0.3711 0.4888 1.0000  164 186 255  #a4baff

31600 K  10deg  0.2483 0.2520  1.380e+17    0.3430 0.5153 1.0000  158 190 255  #9ebeff

31700 K   2deg  0.2510 0.2472  1.231e+17    0.3709 0.4886 1.0000  164 186 255  #a4baff

31700 K  10deg  0.2483 0.2519  1.386e+17    0.3427 0.5151 1.0000  158 190 255  #9ebeff

31800 K   2deg  0.2509 0.2471  1.237e+17    0.3707 0.4884 1.0000  164 186 255  #a4baff

31800 K  10deg  0.2482 0.2519  1.393e+17    0.3425 0.5149 1.0000  158 190 255  #9ebeff

31900 K   2deg  0.2509 0.2471  1.242e+17    0.3704 0.4881 1.0000  164 186 255  #a4baff

31900 K  10deg  0.2482 0.2518  1.399e+17    0.3423 0.5147 1.0000  158 190 255  #9ebeff

32000 K   2deg  0.2509 0.2470  1.248e+17    0.3702 0.4879 1.0000  164 185 255  #a4b9ff

32000 K  10deg  0.2482 0.2518  1.406e+17    0.3420 0.5145 1.0000  158 190 255  #9ebeff

32100 K   2deg  0.2508 0.2470  1.254e+17    0.3700 0.4877 1.0000  164 185 255  #a4b9ff

32100 K  10deg  0.2481 0.2517  1.412e+17    0.3418 0.5143 1.0000  158 190 255  #9ebeff

32200 K   2deg  0.2508 0.2469  1.260e+17    0.3698 0.4875 1.0000  164 185 255  #a4b9ff

32200 K  10deg  0.2481 0.2517  1.419e+17    0.3416 0.5141 1.0000  158 190 255  #9ebeff

32300 K   2deg  0.2508 0.2469  1.265e+17    0.3695 0.4873 1.0000  164 185 255  #a4b9ff

32300 K  10deg  0.2481 0.2516  1.425e+17    0.3413 0.5139 1.0000  158 190 255  #9ebeff

32400 K   2deg  0.2507 0.2468  1.271e+17    0.3693 0.4871 1.0000  164 185 255  #a4b9ff

32400 K  10deg  0.2480 0.2516  1.432e+17    0.3411 0.5137 1.0000  158 190 255  #9ebeff

32500 K   2deg  0.2507 0.2468  1.277e+17    0.3691 0.4869 1.0000  164 185 255  #a4b9ff

32500 K  10deg  0.2480 0.2516  1.438e+17    0.3409 0.5135 1.0000  158 190 255  #9ebeff

32600 K   2deg  0.2507 0.2467  1.283e+17    0.3689 0.4867 1.0000  164 185 255  #a4b9ff

32600 K  10deg  0.2480 0.2515  1.445e+17    0.3407 0.5133 1.0000  158 190 255  #9ebeff

32700 K   2deg  0.2506 0.2467  1.288e+17    0.3687 0.4864 1.0000  163 185 255  #a3b9ff

32700 K  10deg  0.2479 0.2515  1.451e+17    0.3404 0.5132 1.0000  158 190 255  #9ebeff

32800 K   2deg  0.2506 0.2466  1.294e+17    0.3684 0.4862 1.0000  163 185 255  #a3b9ff

32800 K  10deg  0.2479 0.2514  1.458e+17    0.3402 0.5130 1.0000  158 190 255  #9ebeff

32900 K   2deg  0.2506 0.2466  1.300e+17    0.3682 0.4860 1.0000  163 185 255  #a3b9ff

32900 K  10deg  0.2479 0.2514  1.464e+17    0.3400 0.5128 1.0000  158 190 255  #9ebeff

33000 K   2deg  0.2505 0.2465  1.306e+17    0.3680 0.4858 1.0000  163 185 255  #a3b9ff

33000 K  10deg  0.2478 0.2513  1.471e+17    0.3398 0.5126 1.0000  158 190 255  #9ebeff

33100 K   2deg  0.2505 0.2465  1.311e+17    0.3678 0.4856 1.0000  163 185 255  #a3b9ff

33100 K  10deg  0.2478 0.2513  1.477e+17    0.3396 0.5124 1.0000  158 190 255  #9ebeff

33200 K   2deg  0.2505 0.2464  1.317e+17    0.3676 0.4854 1.0000  163 185 255  #a3b9ff

33200 K  10deg  0.2478 0.2513  1.484e+17    0.3393 0.5122 1.0000  157 190 255  #9dbeff

33300 K   2deg  0.2505 0.2464  1.323e+17    0.3674 0.4852 1.0000  163 185 255  #a3b9ff

33300 K  10deg  0.2477 0.2512  1.490e+17    0.3391 0.5120 1.0000  157 190 255  #9dbeff

33400 K   2deg  0.2504 0.2463  1.329e+17    0.3672 0.4850 1.0000  163 185 255  #a3b9ff

33400 K  10deg  0.2477 0.2512  1.497e+17    0.3389 0.5119 1.0000  157 189 255  #9dbdff

33500 K   2deg  0.2504 0.2463  1.334e+17    0.3670 0.4848 1.0000  163 185 255  #a3b9ff

33500 K  10deg  0.2477 0.2511  1.503e+17    0.3387 0.5117 1.0000  157 189 255  #9dbdff

33600 K   2deg  0.2504 0.2463  1.340e+17    0.3668 0.4847 1.0000  163 185 255  #a3b9ff

33600 K  10deg  0.2476 0.2511  1.510e+17    0.3385 0.5115 1.0000  157 189 255  #9dbdff

33700 K   2deg  0.2503 0.2462  1.346e+17    0.3666 0.4845 1.0000  163 185 255  #a3b9ff

33700 K  10deg  0.2476 0.2510  1.516e+17    0.3383 0.5113 1.0000  157 189 255  #9dbdff

33800 K   2deg  0.2503 0.2462  1.352e+17    0.3664 0.4843 1.0000  163 185 255  #a3b9ff

33800 K  10deg  0.2476 0.2510  1.523e+17    0.3381 0.5112 1.0000  157 189 255  #9dbdff

33900 K   2deg  0.2503 0.2461  1.358e+17    0.3662 0.4841 1.0000  163 185 255  #a3b9ff

33900 K  10deg  0.2476 0.2510  1.530e+17    0.3379 0.5110 1.0000  157 189 255  #9dbdff

34000 K   2deg  0.2502 0.2461  1.363e+17    0.3660 0.4839 1.0000  163 185 255  #a3b9ff

34000 K  10deg  0.2475 0.2509  1.536e+17    0.3377 0.5108 1.0000  157 189 255  #9dbdff

34100 K   2deg  0.2502 0.2460  1.369e+17    0.3658 0.4837 1.0000  163 185 255  #a3b9ff

34100 K  10deg  0.2475 0.2509  1.543e+17    0.3375 0.5106 1.0000  157 189 255  #9dbdff

34200 K   2deg  0.2502 0.2460  1.375e+17    0.3656 0.4835 1.0000  163 185 255  #a3b9ff

34200 K  10deg  0.2475 0.2508  1.549e+17    0.3373 0.5105 1.0000  157 189 255  #9dbdff

34300 K   2deg  0.2502 0.2459  1.381e+17    0.3654 0.4833 1.0000  163 185 255  #a3b9ff

34300 K  10deg  0.2474 0.2508  1.556e+17    0.3371 0.5103 1.0000  157 189 255  #9dbdff

34400 K   2deg  0.2501 0.2459  1.386e+17    0.3652 0.4831 1.0000  163 185 255  #a3b9ff

34400 K  10deg  0.2474 0.2508  1.562e+17    0.3369 0.5101 1.0000  157 189 255  #9dbdff

34500 K   2deg  0.2501 0.2459  1.392e+17    0.3650 0.4830 1.0000  163 185 255  #a3b9ff

34500 K  10deg  0.2474 0.2507  1.569e+17    0.3367 0.5100 1.0000  157 189 255  #9dbdff

34600 K   2deg  0.2501 0.2458  1.398e+17    0.3649 0.4828 1.0000  163 185 255  #a3b9ff

34600 K  10deg  0.2473 0.2507  1.575e+17    0.3365 0.5098 1.0000  157 189 255  #9dbdff

34700 K   2deg  0.2500 0.2458  1.404e+17    0.3647 0.4826 1.0000  163 185 255  #a3b9ff

34700 K  10deg  0.2473 0.2506  1.582e+17    0.3363 0.5096 1.0000  157 189 255  #9dbdff

34800 K   2deg  0.2500 0.2457  1.410e+17    0.3645 0.4824 1.0000  163 185 255  #a3b9ff

34800 K  10deg  0.2473 0.2506  1.588e+17    0.3361 0.5095 1.0000  157 189 255  #9dbdff

34900 K   2deg  0.2500 0.2457  1.415e+17    0.3643 0.4822 1.0000  163 185 255  #a3b9ff

34900 K  10deg  0.2473 0.2506  1.595e+17    0.3359 0.5093 1.0000  157 189 255  #9dbdff

35000 K   2deg  0.2500 0.2456  1.421e+17    0.3641 0.4821 1.0000  163 184 255  #a3b8ff

35000 K  10deg  0.2472 0.2505  1.602e+17    0.3357 0.5091 1.0000  157 189 255  #9dbdff

35100 K   2deg  0.2499 0.2456  1.427e+17    0.3639 0.4819 1.0000  163 184 255  #a3b8ff

35100 K  10deg  0.2472 0.2505  1.608e+17    0.3356 0.5090 1.0000  157 189 255  #9dbdff

35200 K   2deg  0.2499 0.2456  1.433e+17    0.3638 0.4817 1.0000  162 184 255  #a2b8ff

35200 K  10deg  0.2472 0.2505  1.615e+17    0.3354 0.5088 1.0000  157 189 255  #9dbdff

35300 K   2deg  0.2499 0.2455  1.439e+17    0.3636 0.4815 1.0000  162 184 255  #a2b8ff

35300 K  10deg  0.2471 0.2504  1.621e+17    0.3352 0.5087 1.0000  157 189 255  #9dbdff

35400 K   2deg  0.2498 0.2455  1.444e+17    0.3634 0.4814 1.0000  162 184 255  #a2b8ff

35400 K  10deg  0.2471 0.2504  1.628e+17    0.3350 0.5085 1.0000  157 189 255  #9dbdff

35500 K   2deg  0.2498 0.2454  1.450e+17    0.3632 0.4812 1.0000  162 184 255  #a2b8ff

35500 K  10deg  0.2471 0.2503  1.634e+17    0.3348 0.5084 1.0000  157 189 255  #9dbdff

35600 K   2deg  0.2498 0.2454  1.456e+17    0.3630 0.4810 1.0000  162 184 255  #a2b8ff

35600 K  10deg  0.2471 0.2503  1.641e+17    0.3346 0.5082 1.0000  156 189 255  #9cbdff

35700 K   2deg  0.2498 0.2454  1.462e+17    0.3629 0.4809 1.0000  162 184 255  #a2b8ff

35700 K  10deg  0.2470 0.2503  1.647e+17    0.3345 0.5080 1.0000  156 189 255  #9cbdff

35800 K   2deg  0.2497 0.2453  1.468e+17    0.3627 0.4807 1.0000  162 184 255  #a2b8ff

35800 K  10deg  0.2470 0.2502  1.654e+17    0.3343 0.5079 1.0000  156 189 255  #9cbdff

35900 K   2deg  0.2497 0.2453  1.473e+17    0.3625 0.4805 1.0000  162 184 255  #a2b8ff

35900 K  10deg  0.2470 0.2502  1.661e+17    0.3341 0.5077 1.0000  156 189 255  #9cbdff

36000 K   2deg  0.2497 0.2452  1.479e+17    0.3624 0.4804 1.0000  162 184 255  #a2b8ff

36000 K  10deg  0.2470 0.2502  1.667e+17    0.3339 0.5076 1.0000  156 189 255  #9cbdff

36100 K   2deg  0.2497 0.2452  1.485e+17    0.3622 0.4802 1.0000  162 184 255  #a2b8ff

36100 K  10deg  0.2469 0.2501  1.674e+17    0.3338 0.5074 1.0000  156 189 255  #9cbdff

36200 K   2deg  0.2496 0.2452  1.491e+17    0.3620 0.4800 1.0000  162 184 255  #a2b8ff

36200 K  10deg  0.2469 0.2501  1.680e+17    0.3336 0.5073 1.0000  156 189 255  #9cbdff

36300 K   2deg  0.2496 0.2451  1.497e+17    0.3619 0.4799 1.0000  162 184 255  #a2b8ff

36300 K  10deg  0.2469 0.2501  1.687e+17    0.3334 0.5071 1.0000  156 189 255  #9cbdff

36400 K   2deg  0.2496 0.2451  1.502e+17    0.3617 0.4797 1.0000  162 184 255  #a2b8ff

36400 K  10deg  0.2468 0.2500  1.693e+17    0.3332 0.5070 1.0000  156 189 255  #9cbdff

36500 K   2deg  0.2496 0.2450  1.508e+17    0.3615 0.4796 1.0000  162 184 255  #a2b8ff

36500 K  10deg  0.2468 0.2500  1.700e+17    0.3331 0.5068 1.0000  156 189 255  #9cbdff

36600 K   2deg  0.2495 0.2450  1.514e+17    0.3614 0.4794 1.0000  162 184 255  #a2b8ff

36600 K  10deg  0.2468 0.2500  1.707e+17    0.3329 0.5067 1.0000  156 189 255  #9cbdff

36700 K   2deg  0.2495 0.2450  1.520e+17    0.3612 0.4792 1.0000  162 184 255  #a2b8ff

36700 K  10deg  0.2468 0.2499  1.713e+17    0.3327 0.5066 1.0000  156 189 255  #9cbdff

36800 K   2deg  0.2495 0.2449  1.526e+17    0.3610 0.4791 1.0000  162 184 255  #a2b8ff

36800 K  10deg  0.2467 0.2499  1.720e+17    0.3326 0.5064 1.0000  156 189 255  #9cbdff

36900 K   2deg  0.2495 0.2449  1.531e+17    0.3609 0.4789 1.0000  162 184 255  #a2b8ff

36900 K  10deg  0.2467 0.2498  1.726e+17    0.3324 0.5063 1.0000  156 189 255  #9cbdff

37000 K   2deg  0.2494 0.2449  1.537e+17    0.3607 0.4788 1.0000  162 184 255  #a2b8ff

37000 K  10deg  0.2467 0.2498  1.733e+17    0.3322 0.5061 1.0000  156 189 255  #9cbdff

37100 K   2deg  0.2494 0.2448  1.543e+17    0.3605 0.4786 1.0000  162 184 255  #a2b8ff

37100 K  10deg  0.2467 0.2498  1.740e+17    0.3321 0.5060 1.0000  156 189 255  #9cbdff

37200 K   2deg  0.2494 0.2448  1.549e+17    0.3604 0.4785 1.0000  162 184 255  #a2b8ff

37200 K  10deg  0.2466 0.2497  1.746e+17    0.3319 0.5058 1.0000  156 188 255  #9cbcff

37300 K   2deg  0.2494 0.2447  1.555e+17    0.3602 0.4783 1.0000  162 184 255  #a2b8ff

37300 K  10deg  0.2466 0.2497  1.753e+17    0.3317 0.5057 1.0000  156 188 255  #9cbcff

37400 K   2deg  0.2493 0.2447  1.561e+17    0.3601 0.4782 1.0000  162 184 255  #a2b8ff

37400 K  10deg  0.2466 0.2497  1.759e+17    0.3316 0.5056 1.0000  156 188 255  #9cbcff

37500 K   2deg  0.2493 0.2447  1.566e+17    0.3599 0.4780 1.0000  162 184 255  #a2b8ff

37500 K  10deg  0.2466 0.2497  1.766e+17    0.3314 0.5054 1.0000  156 188 255  #9cbcff

37600 K   2deg  0.2493 0.2446  1.572e+17    0.3598 0.4779 1.0000  162 184 255  #a2b8ff

37600 K  10deg  0.2465 0.2496  1.772e+17    0.3313 0.5053 1.0000  156 188 255  #9cbcff

37700 K   2deg  0.2493 0.2446  1.578e+17    0.3596 0.4777 1.0000  162 184 255  #a2b8ff

37700 K  10deg  0.2465 0.2496  1.779e+17    0.3311 0.5052 1.0000  156 188 255  #9cbcff

37800 K   2deg  0.2492 0.2446  1.584e+17    0.3595 0.4776 1.0000  162 184 255  #a2b8ff

37800 K  10deg  0.2465 0.2496  1.786e+17    0.3309 0.5050 1.0000  156 188 255  #9cbcff

37900 K   2deg  0.2492 0.2445  1.590e+17    0.3593 0.4774 1.0000  162 184 255  #a2b8ff

37900 K  10deg  0.2465 0.2495  1.792e+17    0.3308 0.5049 1.0000  156 188 255  #9cbcff

38000 K   2deg  0.2492 0.2445  1.595e+17    0.3592 0.4773 1.0000  162 184 255  #a2b8ff

38000 K  10deg  0.2465 0.2495  1.799e+17    0.3306 0.5048 1.0000  156 188 255  #9cbcff

38100 K   2deg  0.2492 0.2445  1.601e+17    0.3590 0.4771 1.0000  162 184 255  #a2b8ff

38100 K  10deg  0.2464 0.2495  1.805e+17    0.3305 0.5046 1.0000  156 188 255  #9cbcff

38200 K   2deg  0.2491 0.2444  1.607e+17    0.3589 0.4770 1.0000  162 184 255  #a2b8ff

38200 K  10deg  0.2464 0.2494  1.812e+17    0.3303 0.5045 1.0000  156 188 255  #9cbcff

38300 K   2deg  0.2491 0.2444  1.613e+17    0.3587 0.4768 1.0000  161 184 255  #a1b8ff

38300 K  10deg  0.2464 0.2494  1.819e+17    0.3302 0.5044 1.0000  156 188 255  #9cbcff

38400 K   2deg  0.2491 0.2444  1.619e+17    0.3586 0.4767 1.0000  161 184 255  #a1b8ff

38400 K  10deg  0.2464 0.2494  1.825e+17    0.3300 0.5042 1.0000  155 188 255  #9bbcff

38500 K   2deg  0.2491 0.2443  1.625e+17    0.3584 0.4766 1.0000  161 184 255  #a1b8ff

38500 K  10deg  0.2463 0.2493  1.832e+17    0.3299 0.5041 1.0000  155 188 255  #9bbcff

38600 K   2deg  0.2491 0.2443  1.630e+17    0.3583 0.4764 1.0000  161 183 255  #a1b7ff

38600 K  10deg  0.2463 0.2493  1.839e+17    0.3297 0.5040 1.0000  155 188 255  #9bbcff

38700 K   2deg  0.2490 0.2443  1.636e+17    0.3581 0.4763 1.0000  161 183 255  #a1b7ff

38700 K  10deg  0.2463 0.2493  1.845e+17    0.3296 0.5038 1.0000  155 188 255  #9bbcff

38800 K   2deg  0.2490 0.2442  1.642e+17    0.3580 0.4761 1.0000  161 183 255  #a1b7ff

38800 K  10deg  0.2463 0.2492  1.852e+17    0.3294 0.5037 1.0000  155 188 255  #9bbcff

38900 K   2deg  0.2490 0.2442  1.648e+17    0.3579 0.4760 1.0000  161 183 255  #a1b7ff

38900 K  10deg  0.2462 0.2492  1.858e+17    0.3293 0.5036 1.0000  155 188 255  #9bbcff

39000 K   2deg  0.2490 0.2442  1.654e+17    0.3577 0.4759 1.0000  161 183 255  #a1b7ff

39000 K  10deg  0.2462 0.2492  1.865e+17    0.3291 0.5035 1.0000  155 188 255  #9bbcff

39100 K   2deg  0.2489 0.2441  1.660e+17    0.3576 0.4757 1.0000  161 183 255  #a1b7ff

39100 K  10deg  0.2462 0.2492  1.872e+17    0.3290 0.5033 1.0000  155 188 255  #9bbcff

39200 K   2deg  0.2489 0.2441  1.666e+17    0.3574 0.4756 1.0000  161 183 255  #a1b7ff

39200 K  10deg  0.2462 0.2491  1.878e+17    0.3288 0.5032 1.0000  155 188 255  #9bbcff

39300 K   2deg  0.2489 0.2441  1.671e+17    0.3573 0.4755 1.0000  161 183 255  #a1b7ff

39300 K  10deg  0.2462 0.2491  1.885e+17    0.3287 0.5031 1.0000  155 188 255  #9bbcff

39400 K   2deg  0.2489 0.2440  1.677e+17    0.3572 0.4753 1.0000  161 183 255  #a1b7ff

39400 K  10deg  0.2461 0.2491  1.891e+17    0.3286 0.5030 1.0000  155 188 255  #9bbcff

39500 K   2deg  0.2489 0.2440  1.683e+17    0.3570 0.4752 1.0000  161 183 255  #a1b7ff

39500 K  10deg  0.2461 0.2490  1.898e+17    0.3284 0.5028 1.0000  155 188 255  #9bbcff

39600 K   2deg  0.2488 0.2440  1.689e+17    0.3569 0.4751 1.0000  161 183 255  #a1b7ff

39600 K  10deg  0.2461 0.2490  1.905e+17    0.3283 0.5027 1.0000  155 188 255  #9bbcff

39700 K   2deg  0.2488 0.2439  1.695e+17    0.3567 0.4749 1.0000  161 183 255  #a1b7ff

39700 K  10deg  0.2461 0.2490  1.911e+17    0.3281 0.5026 1.0000  155 188 255  #9bbcff

39800 K   2deg  0.2488 0.2439  1.701e+17    0.3566 0.4748 1.0000  161 183 255  #a1b7ff

39800 K  10deg  0.2460 0.2490  1.918e+17    0.3280 0.5025 1.0000  155 188 255  #9bbcff

39900 K   2deg  0.2488 0.2439  1.707e+17    0.3565 0.4747 1.0000  161 183 255  #a1b7ff

39900 K  10deg  0.2460 0.2489  1.925e+17    0.3279 0.5024 1.0000  155 188 255  #9bbcff

40000 K   2deg  0.2487 0.2438  1.712e+17    0.3563 0.4745 1.0000  161 183 255  #a1b7ff

40000 K  10deg  0.2460 0.2489  1.931e+17    0.3277 0.5022 1.0000  155 188 255  #9bbcff

# end