User Manual of 7*12W RGBW 4in1 Mini Wash
[image: image1.png]



Product introduction:
Power supply: AC110~230V, 50Hz

Power consumption: 95W

Fiting lamp: 7*12W RGBW LED

DMX-control-channels:12 Channels
Color: RGBW color-mixture

Shuttle: Flash rate: 0-10Hz

Dimmer: 0~100% mechanic dimmer

Maximum PAN-movement: 540

Maximum TILT-movement: 270

Master/Slave ativated by sound at pre-programmed function.
Menu  Display Function:
                [image: image2.wmf]     [image: image3.wmf]    [image: image4.wmf]    [image: image5.wmf]
            Menu  down   up   Enter 
1、ADDR (Address Setting) ：A001-A512 

2、SLND (Master/Slave1/Slave2 Setting)：NAST、SL 1、SL 2

3、SOUN（Sound Active Setting）：0-100

4、AUTO (Auto Mode Setting)：NO/ YES

5、STBY (Turn On/off Mode Setting)：NO/ YES

6、PAN (X Axis Setting)：NO/ YES

7、TILT (Y Axis Setting))：NO/ YES

8、LED (Digital Display On/off  Setting)：ON/OFF

9、DISP (Display Setting)
10：TEST (Function Auto Test)
11、HOUR (Time Record)
12：RSET (Reset) 

Remarks：1.ter It Turn on show：******LED-RGBW******

2.White Balance Adjustment：

     First Step,Press Menu Select Mode.

Second Step,After Press Enter 5 seconds,Enter into White Balance Menu(BALA),LCD Display is Flickering.

Third Step,Press Enter,Enter into White Balance Adjustment.RED(0-255)GREE(0-255)BLUK(0-255)WHIT(0-255)

DMX  Function:
1. Channel 1 Control X Moto(PAN)

CH1：0 Stop, 1-255 Adjust X Motor Running

2. Channle 2 Control X Motor Fine(PAN)

CH2：0 Stop, 1-255 Ajust X Fine Motor

3. Channel 3 Control Y Motor(TILT)

CH3: 0 Stop, 1-255 Ajust Y Fine Motor
4. Channel 4 Control Y Motor Fine Adjust(TILT)

 CH4：0 Stop, 1-255 Ajust Y Fine Motor
5. Channle 5 Control Dimmer

CH5: 0 Shutter, 1-255 Adjust Dimmer, From Dark to Bright

6. Channel 6 Control Red Dimmer （R）

CH6: 0 Shutter, 1-255 Adjust  Red Dimmer, From Dark to Bright

7. Channel 7 Adjust Green Dimmer（G）

CH7: 0 Shutter, 1-255 Adjust  Green Dimmer,From Dark to Bright

8. Channle 8 Adjust Blue Dimmer （B）

CH8: 0 Shutter, 1-255 Adjust  Blue Dimmer,From Dark to Bright

9、Channle 9 Adjust White Dimmer(W)

CH9：0 Shutter, 1-255 Adjust  White Dimmer,From Dark to Bright

10、Channel Adjust RGBW Mixed Color

CH10：0-9 Shutter,0-255 32 kinds Mixed Color Adjusted

11、Channel 11 Control Strobe

CH11：0-9 No Flicker,10-255 Adjust Strobe,From Slow to fast

12:Channel 12 Control Sound Active

CH12：0-127  Sound Active Off,1280255 Sound Active On
_1450203238.unknown

_1450203239.unknown

_1450203237.unknown

_1450203236.unknown

