Punisher
DMX Protocol version 1

 

Destroyer / Punisher DMX Protocol multi-channel mode (Implemented from software version 1.0)

Destroyer / Punisher DMX Protocol single-channel mode (Implemented from software version 1.0)

 

 

	Punisher DMX Protocols multi-channel mode version 1
(Implemented from software version 1.0) 

	DMX Channel Offset
	DMX Value
	Description

	0
	0 - 11
12 - 127
128 - 255
	Lamp ON/OFF
Lamp OFF 
Lamp ON 
Gobo strobe: slow -> fast

	1
	0 - 11
12 - 127
128 - 255
	Trigger Mode 
Off 
Music triggered 
Controller triggered - triggers when DMX level crosses 195

	2
	0 - 91
92 - 98 
99 - 189 
190 - 231 
232 - 255
	Parabol Speed

NOTE:
Rotation time equal to trigger decay if trigger mode
set to music or controller 

Rotation CW, fast -> slow 
Stop 
Rotation CCW, slow -> fast 
Random CW/CCW slow (music/controller triggered) 
Random CW/CCW fast (music/controller triggered)

	3
	0 - 4
5 - 252 
253 - 255
	Trigger Decay
0 seconds 
0.2 to 9.9 seconds 
Random (music/controller triggered)

	4
	0 - 5
6 - 20 
21 - 36 
37 -52 
53 - 68 
69 - 84 
85 - 100 
101 - 116 
117 - 132 
133 - 148 
149 - 164 
165 - 180 
181 - 196 
197 - 244 
245 - 255
	Color/Gobo Wheel
Blackout 
Position 1 
Position 2 
Position 3 
Position 4 
Position 5 
Position 6 
Position 7 
Position 8 
Position 9 
Position 10 
Position 11 
Position 12 
No effect 
Random (music/controller triggered)

	5
	0-4
5-252 
253-255
	Shaker
Off 
Slow -> fast 
Random (music/controller triggered)


 

 

 

	Punisher DMX Protocols single-channel mode version 1
(Implemented from software version 1.0) 

	DMX Channel Offset
	DMX Value
	Trigger Decay

	1
	0 - 50 
51 - 101 
102 - 152 
153 - 203 
204 - 255
	Lamp OFF
2.0 seconds (slow) 
1.0 seconds (medium) 
0.2 seconds (fast) 
0.2 - 2.5 seconds (random)


 

