Single head RGB Scanning+Pattern 2 in 1 Laser Light

User’s Manual

Thank you very much for purchasing single head RGB scanning+pattern 2 in 1 laser light. For your personal safety and better use of this product, please read this manual carefully before use and follow the instructions to avoid personal injury and damage the lamp.

ENTER

UP

DOWN

MENU

● Packing List

Please check if you have the following items when you open the light box:

Single head RGB scanning+pattern 2 in 1 laser light   ······· one

Manual                                      ······· one

● Technical Parameters：

Power supply：AC100～240V ，50Hz/60Hz

Laser power：500mwRGB scanning+R200mw,G100mw pattern

Chassis material：iron

Scanning system: new high-speed large-angle precision motor

Power connection: power cord

Signal connection: signal input line/output line

Control channel: 10 DMX-512 signal channels

Control mode: DMX-512 signal control, voice control, auto and master-slave mode

Cooling system: fan forced cooling system

Working environment: indoor

Packing mode: carton

Product size： 19.5*14*7.2cm

Inner box size：24*19*14cm

Carton size：74*50*40cm (20个/carton）

N.W：1.45KG

G.W：1.7KG

● Instructions for use:

After opening the box, carefully check whether the lamp is damaged by transportation, such as whether the screw is loose, whether the lens is broken or not. Please check before powering up. Check that the lamp is placed correctly and securely, and check that the power supply voltage is the same as the product.

● Display Function：this product adopts display mode operation and DMX mode operation，the specific settings are as follow：

| NO. | Display | Function Instruction |
| --- | --- | --- |
| 1 | A001-A512 | DMX512 address code setting |
| 2 | Au01-Au04 | Auto mode、4 modes in total |
| 3 | So01-So04 | Sound mode、4 modes in total |
| 4 | SE01-SE09 | Sound control sensitivity |
| 5 | CL01-CL03 | Color selection，CL01：single color， CL02：RG， CLO3：RGY，CLO4：RGB |

●DMX channel function: This product uses standard DMX-512 signal, a total of 10 control channels, the control content is as follows：

| Channel | Channel Instruction | Value range | Function Instruction |
| --- | --- | --- | --- |
| CH1 | Mode selection | 0-63 | Closed light mode |
|  |  | 64-127 | Manual mode |
|  |  | 128-191 | Auto mode |
|  |  | 192-255 | Sound mode |
| CH2 | Closed light mode | No | No |
|  | Manual mode | 0-255 | 51 patterns to choose |
|  | Auto mode | 0-63 | Auto mode 1 |
|  |  | 64-127 | Auto mode 2 |
|  |  | 128-191 | Auto mode 3 |
|  |  | 192-255 | Auto mode 4 |
|  | Sound mode | 0-63 | Sound mode 1 |
|  |  | 64-127 | Sound mode 2 |
|  |  | 128-191 | Sound mode 3 |
|  |  | 192-255 | Sound mode 4 |
| CH3 | Angle control | 0-127 | Rotation angle selection |
|  |  | 128-191 | Positive rotation speed selection, the larger the value, the faster the speed |
|  |  | 192-255 | Anti-rotation speed selection, the larger the value, the faster the speed |
| CH4 | Horizontal angle | 0-127 | Horizontal flip position selection |
|  |  | 128-255 | Horizontal flip speed selection, the larger the value, the faster the speed |
| CH5 | Vertical angle | 0-127 | Vertical flip position selection |
|  |  | 128-255 | Vertical flip speed selection, the larger the value, the faster the speed |
| CH6 | horizontal position | 0-127 | Horizontal position selection |
|  |  | 128-255 | Horizontal movement speed selection, the larger the value, the faster the speed |
| CH7 | Vertical position | 0-127 | Vertical position selection |
|  |  | 128-255 | Vertical movement speed selection, the larger the value, the faster the speed |
| CH8 | Pattern size control | 0-63 | Fixed size, the larger the value, the smaller the pattern |
|  |  | 64-127 | Choose from small to large speed, the larger the value, the faster the speed |
|  |  | 128-191 | Choose from large to small speed, the larger the value, the faster the speed |
|  |  | 192-255 | Size zoom speed selection, the larger the value, the faster the speed |
| CH9 | Color control | 0-63 | Monochrome color selection |
|  |  | 64-127 | Color mixing |
|  |  | 128-191 | Monochrome Auto |
|  |  | 192-255 | Auto |
| CH10 | Code control | 0-127 | The pattern has dots and lines |
|  |  | 128-255 | Pattern with dots, wireless strips |

● Maintenance:

1. Try to prevent the artificial laying of dust, dirt and smoke oil, or even flow into the lamp body, and use the environment to keep the laser performance device as clean as possible;

2. Please use a professional glass cleaner to clean the lens regularly every month to ensure the maximum laser light output and extend the life of the light source.

3. Do not face the semiconductor light-emitting body for a long time, so as not to hurt the eyes.